SailboatServer — GL.iNet & Portal Update Log

Session: March 7, 2026  ·  SV-Esperanza  ·  Pi 4 Build

# Section 1 — GL.iNet GL-AR300M Router Setup

The GL.iNet 300M router was configured to serve as the boat's Wi-Fi access point and internet gateway. On the boat it will sit between the Starlink ethernet adapter (WAN) and the crew's devices (LAN/Wi-Fi). At home it is used for development with house ethernet on WAN.

## 1.1 Hardware

| Model | GL.iNet GL-AR300M (300M) |
| --- | --- |
| Firmware | OpenWrt |
| Radio | 2.4GHz only |
| WAN port | House ethernet (dev) / Starlink (boat) |
| LAN port | SailboatServer Pi ethernet (eth0) |
| LAN IP | 192.168.8.1 |
| SSID | SV-Esperanza |
| Wi-Fi Password | Bund3r!8!8 |
| Root password | Bund3r!8!8 |

## 1.2 Setup Steps Completed

- Factory reset router

- Set root password via GL.iNet admin UI

- Enabled SSH access

- Connected house ethernet to WAN port, verified internet (ping 8.8.8.8)

- Ran opkg update

- Installed nlbwmon for bandwidth monitoring

- Configured Wi-Fi: SSID=SV-Esperanza, HT20, channel 6, guest network disabled

- Confirmed WAN interface is eth0 (reads /proc/net/dev for traffic stats)

- Connected SailboatServer Pi ethernet cable to GL.iNet LAN port

- Pi obtained DHCP address 192.168.8.214 from GL.iNet

## 1.3 SSH Access from Mac

The GL.iNet uses older SSH key algorithms. Add this to ~/.ssh/config on your Mac:

Host 192.168.8.1

HostKeyAlgorithms +ssh-rsa

PubkeyAcceptedAlgorithms +ssh-rsa

User root

Then SSH normally:

ssh 192.168.8.1

## 1.4 SSH Key from Pi to Router

Passwordless SSH was configured from the Pi to the router so relay_server.py can poll traffic stats without a password:

ssh-keygen -t rsa -b 2048 -f ~/.ssh/id_rsa_router -N ""

ssh-copy-id -o HostKeyAlgorithms=+ssh-rsa -o PubkeyAcceptedAlgorithms=+ssh-rsa \

-i ~/.ssh/id_rsa_router.pub root@192.168.8.1

Pi SSH config at ~/.ssh/config:

Host 192.168.8.1

IdentityFile ~/.ssh/id_rsa_router

HostKeyAlgorithms +ssh-rsa

PubkeyAcceptedAlgorithms +ssh-rsa

User root

## 1.5 Network Topology

### Development (Home)

House ethernet → GL.iNet WAN (eth0, 192.168.50.213) → GL.iNet LAN → Pi (192.168.8.214)

Pi also connected to home Wi-Fi (TheGrid_5G, 192.168.50.49) as fallback — lower priority route (metric 600 vs 100).

### Boat (Production)

Starlink ethernet adapter → GL.iNet WAN → GL.iNet LAN → Pi (eth0, DHCP from GL.iNet)

Crew devices connect to SV-Esperanza Wi-Fi (192.168.8.x range).

SBS hotspot (SSID: SBS, 192.168.4.1) remains as fallback when GL.iNet/Starlink is off.

# Section 2 — relay_server.py New Endpoints

The following endpoints were added to /home/pi/relay_server.py during this session. They were injected via a Python heredoc script into the file before the if __name__ == '__main__': block.

## 2.1 Starlink Status Endpoint

GET /starlink/status — SSHes to the GL.iNet router, reads /proc/net/dev and ip route show default, returns connection status, byte counters, WAN IP, relay CH8 state, and daily GB accumulator.

Key constants in relay_server.py:

| ROUTER_IP | 192.168.8.1 |
| --- | --- |
| ROUTER_USER | root |
| ROUTER_IFACE | eth0 |

Example response:

{"connected": true, "daily_gb": 0.9, "relay_on": false,

"rx_bytes": 5179471, "tx_bytes": 271599, "wan_ip": "192.168.50.213"}

## 2.2 Router Reboot Endpoint

POST /router/reboot — SSHes to the router and runs reboot. Used by the Reboot Router button in the portal relay drawer.

## 2.3 System Endpoints (previously added)

- POST /system/reboot — runs sudo reboot

- POST /system/shutdown — runs sudo shutdown now

- POST /system/network/hotspot — runs /home/pi/switch-to-hotspot.sh

- POST /system/network/wifi — reads JSON body {network: name}, calls switch-to-wifi.sh

# Section 3 — Portal (index.html) Updates

## 3.1 Branding Update

- Header h1 changed from EwokPrep to SV-Esperanza

- Footer changed from EWOKPREP to SAILBOATSERVER

- All body text references to EwokPrep updated to SV-Esperanza or SBS

- Wi-Fi network references updated from EwokPrep to SBS

- SSH reconnect hostname updated from ewokprep.local to sailboatserver.local

- Default Wi-Fi network in switchNetwork() prompt changed from TheGrid_5G to SV-Esperanza

## 3.2 Starlink Status in Header Bar

The header bar now shows live Starlink connection status polled every 5 seconds from /starlink/status:

- 🛰 ONLINE (green) / OFFLINE (red) status indicator

- ↓XX ↑XX Mbps speeds calculated from byte counter deltas between polls

- Speeds only show when connected is true

- Speed calculation requires two samples — shows after first 5-second poll cycle

Known behavior: speeds show 0.0 when no traffic is flowing through the GL.iNet WAN. On the boat with Starlink as the only internet source, real speeds will be visible whenever crew devices are active.

## 3.3 Bug Fixes

- fetchRelayStates was defined in bottom script block — moved all relay functions to head script block so toggleRelayDrawer() can reference them

- Bottom script block had duplicate truncated relay function definitions — removed duplicates, kept only search listener and renderRelays(null) pre-render call

- slStatusPill element reference was crashing updateStarlinkStatus() before DOM updates — removed reference to non-existent element

- Speed display was referencing slDownload/slUpload elements from deleted metrics strip — removed those references

- Speed comparison dl < 0 compared string to number — fixed to parseFloat(dl) < 0

## 3.4 Portal Deployment

The canonical portal source is maintained locally at /mnt/user-data/outputs/index.html and deployed to the Pi with:

scp ~/Downloads/index.html pi@sailboatserver.local:/home/pi/index.html

ssh pi@sailboatserver.local "sudo cp /home/pi/index.html /var/www/html/index.html"

# Section 4 — Kiwix library.xml Rebuild

Kiwix failed to start after a reboot because /home/pi/zims/library.xml was missing (deleted or never created). All ZIM files were intact. Fix:

/home/pi/kiwix-tools_linux-aarch64-3.8.2/kiwix-manage \

/home/pi/zims/library.xml add /home/pi/zims/*.zim

sudo systemctl restart kiwix

If this happens again after future reboots, run the same kiwix-manage command. Consider adding it to a boot script or cron job on the Pi 5.

# Section 5 — Current System State

## 5.1 Services

| kiwix | active (running) — port 8080 |
| --- | --- |
| nginx | active (running) — port 80 |
| relay | active (running) — port 8081 |

## 5.2 Pi Network Interfaces

| eth0 | 192.168.8.214 (DHCP from GL.iNet) — metric 100 (primary) |
| --- | --- |
| wlan0 | 192.168.50.49 (home Wi-Fi TheGrid_5G) — metric 600 (fallback) |
| hotspot | 192.168.4.1 (SBS hotspot — available via switch-to-hotspot.sh) |

## 5.3 Portal Feature Status

| Header branding | SV-Esperanza ✅ |
| --- | --- |
| Starlink ONLINE/OFFLINE | Working ✅ |
| Starlink speed meter | Working ✅ (shows 0.0 when idle) |
| Relay drawer — 8 channels | Working ✅ |
| Starlink power on/off (CH8) | Working ✅ |
| Router reboot button | Working ✅ |
| Kiwix content library | Working ✅ (~90GB ZIM files) |
| Network switching buttons | Working ✅ |
| Restart / Shutdown Pi | Working ✅ |

## 5.4 Remaining Pi 4 Tasks

- None — Pi 4 SBS is feature complete for current scope

- library.xml rebuild command documented above for future reference

- Pi 5 hardware purchase pending — see SailboatServer_Pi5_Build_Guide.docx for migration plan

— End of Session Log —