SailboatServer (SBS)

Raspberry Pi 4 Offline Knowledge Server

Pacific Passage Edition  ·  Build Documentation

# 1. Project Overview

SailboatServer (SBS) is a Raspberry Pi 4-based offline knowledge server built for Pacific Ocean sailing passages. It provides a complete reference library, PDF documents, and 12V device control — all accessible via a built-in Wi-Fi hotspot with no internet connection required.

## Access

- Wi-Fi Hotspot SSID: SBS

- Portal URL: http://192.168.4.1

- Kiwix Library: http://192.168.4.1:8080

- Relay Controls: http://192.168.4.1:8081

- SSH: ssh pi@192.168.4.1 (when on SBS hotspot)

- SSH: ssh pi@sailboatserver.local (when on home network via ethernet)

## Hardware

- Raspberry Pi 4 Model B Rev 1.5

- 256GB SD card (upgradeable to SSD)

- Waveshare RPi Relay Board (B) — 8 channel relay HAT

- Ethernet cable for home network access during maintenance

# 2. System Setup

## Operating System

- Raspberry Pi OS Lite (64-bit) — Debian GNU/Linux 13 (trixie)

- Kernel: Linux 6.12.62+rpt-rpi-v8 (arm64)

- Hostname: SailboatServer

- Username: pi

- SSH enabled, Wi-Fi pre-configured via Raspberry Pi Imager

## Key Directories

| Path | Purpose |
| --- | --- |
| /home/pi/zims/ | Kiwix ZIM content files |
| /var/www/html/ | Nginx web portal |
| /var/www/html/pdfs/ | PDF documents |
| /home/pi/relay_control.py | Relay GPIO library |
| /home/pi/relay_server.py | Flask relay web server |
| /home/pi/build-library.sh | Kiwix library rebuild script |
| /home/pi/switch-to-wifi.sh | Switch to home Wi-Fi mode |
| /home/pi/switch-to-hotspot.sh | Switch back to hotspot mode |
| /boot/firmware/user-data | Cloud-init config (hostname etc) |

## Services

| Service | Port | Description |
| --- | --- | --- |
| nginx | 80 | Web portal & PDF serving |
| kiwix | 8080 | Offline content library |
| relay | 8081 | Flask relay controller |

## Kiwix Installation

Kiwix-tools 3.8.2 installed from:

https://download.kiwix.org/release/kiwix-tools/kiwix-tools_linux-aarch64-3.8.2.tar.gz

Binaries copied to /usr/local/bin/

Systemd service: /etc/systemd/system/kiwix.service (port 8080, auto-start enabled)

Library auto-rebuild: /home/pi/build-library.sh (cron at 2am daily)

# 3. Network Configuration

## Two-Mode Network Strategy

The Pi 4 has a single Wi-Fi radio which cannot simultaneously run a hotspot and connect to home Wi-Fi. Two modes are used:

- Hotspot Mode (default/deployment): SSID 'SBS', IP 192.168.4.1 — for use at sea

- Wi-Fi Mode (maintenance): Connects to home/marina network for downloading content

- Ethernet: Always available for SSH and internet access when plugged in

## Switching Networks

Switch to Wi-Fi for downloading (SSH in first via hotspot or ethernet):

~/switch-to-wifi.sh NetworkName

# Example: ~/switch-to-wifi.sh TheGrid_5G

# Example: ~/switch-to-wifi.sh MarinaWifi

Switch back to hotspot when done:

~/switch-to-hotspot.sh

## NetworkManager Connection Names

- Hotspot: SBSHotspot (UUID: 79638dc0-2b91-4a10-be4e-d1ebf7e4d86c)

- Home Wi-Fi: netplan-wlan0-TheGrid_5G

- Ethernet: netplan-eth0

# 4. Content Library

All ZIM files stored in /home/pi/zims/ — total approximately 90GB

## Core Reference (~53GB)

| File | Size | Description |
| --- | --- | --- |
| wikipedia_en_all_nopic_2025-12.zim | 48GB | Wikipedia (no images) |
| wikibooks_en_all_maxi_2026-01.zim | 5.1GB | Wikibooks |
| wikivoyage_en_all_nopic_2025-12.zim | 229MB | Wikivoyage travel guide |

## Medical & Emergency (~1.3GB)

| File | Size | Description |
| --- | --- | --- |
| zimgit-medicine_en_2024-08.zim | 67MB | Field & emergency medicine |
| medicalsciences.stackexchange.com_en_all_2026-02.zim | 58MB | Medical Sciences Q&A |
| zimgit-post-disaster_en_2024-05.zim | 615MB | Post-disaster resources |

## Sailing & Navigation (~600MB)

| File | Size | Description |
| --- | --- | --- |
| astronomy.stackexchange.com_en_all_2026-02.zim | 187MB | Astronomy & celestial nav |
| zimgit-knots_en_2024-08.zim | 27MB | Illustrated knots library |
| travel.stackexchange.com_en_all_2026-02.zim | 439MB | Travel & passage planning |
| outdoors.stackexchange.com_en_all_2026-02.zim | 136MB | Outdoors Q&A |
| sports.stackexchange.com_en_all_2026-02.zim | 42MB | Sports Q&A |

## Repair & Maintenance (~6.5GB)

| File | Size | Description |
| --- | --- | --- |
| ifixit_en_all_2025-12.zim | 3.4GB | iFixit repair manuals |
| diy.stackexchange.com_en_all_2026-02.zim | 1.9GB | DIY home improvement |
| mechanics.stackexchange.com_en_all_2026-02.zim | 324MB | Motor vehicle mechanics |
| engineering.stackexchange.com_en_all_2026-02.zim | 243MB | Engineering Q&A |

## Food & Water (~1.2GB)

| File | Size | Description |
| --- | --- | --- |
| cooking.stackexchange.com_en_all_2026-02.zim | 227MB | Cooking Q&A |
| gardening.stackexchange.com_en_all_2026-02.zim | 883MB | Gardening Q&A |
| zimgit-food-preparation_en_2025-04.zim | 93MB | Food prep for preppers |
| zimgit-water_en_2024-08.zim | 20MB | Water treatment |

## Science & Nature (~4.9GB)

| File | Size | Description |
| --- | --- | --- |
| wikispecies_en_all_maxi_2026-01.zim | 3.1GB | All species catalogue |
| biology.stackexchange.com_en_all_2026-02.zim | 403MB | Biology Q&A |
| chemistry.stackexchange.com_en_all_2026-02.zim | 397MB | Chemistry Q&A |

## Project Gutenberg (~21.4GB)

| File | Size | Description |
| --- | --- | --- |
| gutenberg_en_lcc-r_2025-12.zim | 1.8GB | Medicine |
| gutenberg_en_lcc-s_2025-12.zim | 4.2GB | Agriculture |
| gutenberg_en_lcc-t_2025-12.zim | 12GB | Technology |
| gutenberg_en_lcc-u_2025-12.zim | 1.2GB | Military science |
| gutenberg_en_lcc-v_2025-12.zim | 1.3GB | Naval science |

## Other (~600MB)

| File | Size | Description |
| --- | --- | --- |
| ham.stackexchange.com_en_all_2026-02.zim | 73MB | Amateur radio Q&A |
| homebrew.stackexchange.com_en_all_2026-02.zim | 36MB | Homebrewing Q&A |
| security.stackexchange.com_en_all_2026-02.zim | 420MB | Information security |
| sustainability.stackexchange.com_en_all_2026-02.zim | 26MB | Sustainable living |

# 5. PDF Library

All PDFs stored in /var/www/html/pdfs/ and accessible via the portal.

## Navigation Documents

| File | Size | Description |
| --- | --- | --- |
| Bowditch_Vol_1.pdf | 167MB | American Practical Navigator Vol 1 |
| Bowditch_Vol_2.pdf | 31MB | American Practical Navigator Vol 2 |
| Bowditch_Vol_1_LoRes.pdf | 43MB | Bowditch Vol 1 low-res |
| Bowditch_Vol_2_LoRes.pdf | 11MB | Bowditch Vol 2 low-res |
| V1_Pub229Vol1.pdf – V6_Pub229Vol6.pdf | 4.9–13MB each | NGA Sight Reduction Tables (all 6 volumes) |
| USCG_Navigation_Rules_COLREGS.pdf | 1.3MB | COLREGS Navigation Rules |
| navigation_mode.pdf | 233KB | Navigation mode reference |

## Equipment & Survival

| File | Size | Description |
| --- | --- | --- |
| GX2000-GX2150-Manual.pdf | 3.5MB | VHF radio manual |
| victron-raspi-stetson.pdf | 700KB | Victron power & solar reference |
| Slow+Sand+Filter_6.26.2021.pdf | 4.4MB | Slow sand water filtration guide |

# 6. Relay Controller

## Hardware

Waveshare RPi Relay Board (B) — 8 channel relay HAT. Low-active GPIO (LOW = relay ON, HIGH = relay OFF).

## GPIO Pin Mapping

| Relay | GPIO Pin | Device | Notes |
| --- | --- | --- | --- |
| CH1 | GPIO26 | Cabin Lights |  |
| CH2 | GPIO20 | Navigation Lights |  |
| CH3 | GPIO21 | Anchor Light |  |
| CH4 | GPIO5 | Bilge Pump |  |
| CH5 | GPIO6 | Water Pump |  |
| CH6 | GPIO13 | Ventilation Fan |  |
| CH7 | GPIO19 | Instruments/Electronics |  |
| CH8 | GPIO16 | Spare |  |

## Software

- GPIO library: lgpio (required for Raspberry Pi OS Bookworm/Trixie)

- Web server: Flask (python3-flask)

- CORS: python3-flask-cors

- Service: /etc/systemd/system/relay.service (auto-starts on boot)

- Control files: /home/pi/relay_control.py and /home/pi/relay_server.py

## API Endpoints

| Endpoint | Method | Description |
| --- | --- | --- |
| /relay/states | GET | Returns JSON of all relay states |
| /relay/<ch>/on | POST | Turn relay channel on |
| /relay/<ch>/off | POST | Turn relay channel off |
| /relay/all/off | POST | Turn all relays off |

# 7. Web Portal

Custom dark-theme portal served by Nginx at http://192.168.4.1 (port 80).

## Features

- Search filter — filters all content cards in real time

- Emergency quick-access strip (red) — direct links to critical resources

- Boat Controls drawer — slides down from header, controls all 8 relays inline

- 11 content category cards with direct Kiwix links

- Navigation PDF section with all Bowditch and Pub 229 volumes

- Celestial navigation section

- Weather & passage planning reference section

- Admin instructions section with all maintenance commands

- Clock showing local time with UTC offset

## Link Format

All Kiwix content links use onclick navigation to avoid cross-origin browser blocking:

onclick="kiwix('content/wikipedia_en_all_nopic_2025-12')"

The kiwix() function builds the full URL using window.location.hostname so it works on both the hotspot (192.168.4.1) and home network (sailboatserver.local).

# 8. Maintenance Procedures

## Adding New ZIM Content

1. SSH into the Pi (connect to SBS hotspot first or use ethernet):

ssh pi@192.168.4.1  # via hotspot

ssh pi@sailboatserver.local  # via ethernet/home network

2. Switch to Wi-Fi for downloading:

~/switch-to-wifi.sh TheGrid_5G

3. Reconnect via home network and download:

cd /home/pi/zims

wget -b -o filename.log https://download.kiwix.org/zim/...

tail -f filename.log  # monitor progress

4. Rebuild library and switch back to hotspot:

~/build-library.sh

sudo systemctl restart kiwix

~/switch-to-hotspot.sh

## Adding New PDFs

From your Mac (while on home network):

scp file.pdf pi@sailboatserver.local:/home/pi/pdfs/

Then on Pi:

sudo cp /home/pi/pdfs/*.pdf /var/www/html/pdfs/

## Updating the Portal

Edit directly on the Pi:

sudo nano /var/www/html/index.html

Or transfer from Mac:

scp index.html pi@sailboatserver.local:/home/pi/index.html

sudo cp /home/pi/index.html /var/www/html/index.html

## Checking Disk Space

df -h /

ls -lh /home/pi/zims/*.zim

## Service Management

sudo systemctl status kiwix nginx relay

sudo systemctl restart kiwix

sudo systemctl restart relay

sudo systemctl restart nginx

## Safe Shutdown

sudo shutdown now

Wait for the green activity light to stop blinking before unplugging power.

# 9. Pending / Future Work

- Nav Pi — separate Raspberry Pi with OpenCPN chart plotter, MacArthur HAT, GPS

- Power strategy — battery bank, solar panel, 12V adapter for ship's power

- USB Wi-Fi adapter — would allow simultaneous hotspot + home Wi-Fi without switching

- SSD upgrade — replace 256GB SD card with external SSD for reliability

- Additional PDFs — Chapman's Piloting, Pacific Crossing Guide, celestial nav workbooks

- Relay wiring — physical 12V wiring to boat systems for relay channels

# 10. Quick Reference

## Key IP Addresses & URLs

| Resource | Address |
| --- | --- |
| Portal (hotspot) | http://192.168.4.1 |
| Portal (home network) | http://sailboatserver.local |
| Kiwix library | http://192.168.4.1:8080 |
| Relay controller | http://192.168.4.1:8081 |
| SSH (hotspot) | ssh pi@192.168.4.1 |
| SSH (home network) | ssh pi@sailboatserver.local |
| Wi-Fi hotspot SSID | SBS |

## Most Used Commands

| Command | Purpose |
| --- | --- |
| ~/switch-to-wifi.sh NetworkName | Switch to Wi-Fi for downloading |
| ~/switch-to-hotspot.sh | Switch back to hotspot mode |
| ~/build-library.sh | Rebuild Kiwix library after adding ZIMs |
| sudo systemctl restart kiwix | Restart Kiwix |
| sudo systemctl restart relay | Restart relay controller |
| df -h / | Check disk space |
| sudo shutdown now | Safe shutdown |
| sudo reboot | Reboot |