SailboatServer (SBS)

Pi 5 Build, Migration & Navigation Setup Guide

Pacific Passage Edition  ·  March 2026

# 1. Project Overview

This document covers the migration of SailboatServer (SBS) from a Raspberry Pi 4 to a Raspberry Pi 5, the addition of OpenCPN navigation software with Pacific chart folios, NMEA device integration via the MacArthur HAT, and a unified boat dashboard web interface accessible from any device on the SBS hotspot.

## System Architecture

| Component | Role |
| --- | --- |
| SailboatServer (SBS) | Offline knowledge library, relay control, navigation, NMEA integration |
| Raspberry Pi 5 8GB | Main computer running all services |
| GeeekPi P33 NVMe HAT | 1TB NVMe SSD storage via PCIe, PoE power option |
| MacArthur HAT | NMEA 0183/2000/SeaTalk1 integration, IMU, barometer, GPS |
| GeeekPi UPS Gen 6 | Clean shutdown on power loss, battery backup |
| Waveshare Relay Board (B) | 8-channel 12V device control |
| SBS Hotspot (SSID: SBS) | Wi-Fi access point for all crew devices |

## HAT Stack Order (bottom to top)

| Position | Component | Connection Method |
| --- | --- | --- |
| Bottom (under Pi 5) | GeeekPi UPS Gen 6 | Pogo pins to Pi 5 underside |
| Pi 5 | Raspberry Pi 5 8GB | — |
| HAT 1 | GeeekPi P33 NVMe + PoE HAT | GPIO header + PCIe FFC cable |
| HAT 2 | MacArthur HAT | GPIO passthrough from P33 |
| HAT 3 | Waveshare Relay Board (B) | GPIO passthrough from MacArthur HAT |

## GPIO & I2C Conflict Analysis

All components have been verified for GPIO and I2C conflicts:

| Component | GPIO Pins | I2C Address | Conflicts |
| --- | --- | --- | --- |
| Waveshare Relay Board | 5,6,13,16,19,20,21,26 | None | None |
| MacArthur HAT | 2,3 (I2C), 8-11 (SPI), 14,15 (UART) | 0x68, 0x76/77, 0x40 | None |
| GeeekPi UPS Gen 6 | None (pogo pins) | 0x36 | None |
| GeeekPi P33 NVMe HAT | Passthrough only | None | None |

# 2. Hardware Shopping List

Items already owned are marked. Remaining items to purchase:

## Items to Purchase

| Item | Est. Price | Where to Buy | Notes |
| --- | --- | --- | --- |
| Raspberry Pi 5 8GB | ~$120-130 | PiShop.us / Micro Center | 8GB RAM for running all services simultaneously |
| GeeekPi P33 NVMe + PoE HAT | ~$45-55 | Amazon | Includes active cooler, GPIO passthrough, supports 2230/2242/2260/2280 |
| 500GB NVMe M.2 2280 SSD | ~$50-60 | Amazon / Micro Center | Kingston NV3 or WD Blue SN580 recommended |
| Waveshare Relay Board (B) 8ch | ~$25 | Amazon / Waveshare | Same model as current SBS Pi 4 |

## Items Already Owned

| Item | Notes |
| --- | --- |
| MacArthur HAT | With power module and ICM-20948 IMU |
| GeeekPi UPS Gen 6 | Includes active cooler and pogo pin connector |
| GPS Module | Connects to MacArthur HAT UART0 |
| Raspberry Pi 4 (current SBS) | Keep as spare/backup |

Estimated total spend: ~$240-270. Enclosure will be 3D printed (PETG recommended for marine environment).

# 3. Phase 1 — Raspberry Pi 5 Base Setup

## 3.1 Flash OS

Use Raspberry Pi Imager on your Mac:

- OS: Raspberry Pi OS (64-bit) — Full desktop version required for OpenCPN

- Enable SSH in advanced settings

- Set hostname: SailboatServer

- Set username: pi

- Configure Wi-Fi: TheGrid_5G (for initial setup)

## 3.2 Initial Boot & Update

ssh pi@sailboatserver.local

sudo apt update && sudo apt upgrade -y

sudo apt install -y git python3-pip python3-flask python3-flask-cors python3-lgpio nginx

## 3.3 Enable PCIe for NVMe

The P33 NVMe HAT requires PCIe to be enabled on the Pi 5:

sudo nano /boot/firmware/config.txt

Add at the bottom:

dtparam=pciex1

dtparam=pciex1_gen=3

Save and reboot:

sudo reboot

## 3.4 Format & Mount NVMe

After reboot, verify NVMe is detected:

lsblk

You should see nvme0n1. Partition and format it:

sudo fdisk /dev/nvme0n1

In fdisk: press n (new partition), p (primary), 1 (partition 1), accept defaults, w (write).

sudo mkfs.ext4 /dev/nvme0n1p1

sudo mkdir /mnt/nvme

sudo mount /dev/nvme0n1p1 /mnt/nvme

Add to fstab for auto-mount on boot:

echo '/dev/nvme0n1p1 /mnt/nvme ext4 defaults 0 2' | sudo tee -a /etc/fstab

## 3.5 Move Content to NVMe

Create directory structure on NVMe:

sudo mkdir -p /mnt/nvme/zims /mnt/nvme/pdfs /mnt/nvme/charts

sudo chown -R pi:pi /mnt/nvme

# 4. Phase 2 — Migrate Content from Pi 4

## 4.1 Transfer ZIM Files

Both Pis need to be on the same network. Transfer ZIMs directly from Pi 4 to Pi 5 via rsync — much faster than re-downloading:

# Run this on the Pi 5

rsync -avz --progress pi@ewokprep.local:/home/pi/zims/ /mnt/nvme/zims/

This will take a while — approximately 90GB to transfer. On gigabit ethernet expect 30-60 minutes.

## 4.2 Transfer PDFs

rsync -avz pi@ewokprep.local:/var/www/html/pdfs/ /mnt/nvme/pdfs/

## 4.3 Transfer Portal & Config Files

rsync -avz pi@ewokprep.local:/var/www/html/index.html /home/pi/

rsync -avz pi@ewokprep.local:/home/pi/relay_control.py /home/pi/

rsync -avz pi@ewokprep.local:/home/pi/relay_server.py /home/pi/

rsync -avz pi@ewokprep.local:/home/pi/build-library.sh /home/pi/

rsync -avz pi@ewokprep.local:/home/pi/switch-to-wifi.sh /home/pi/

rsync -avz pi@ewokprep.local:/home/pi/switch-to-hotspot.sh /home/pi/

## 4.4 Install Kiwix

Download and install Kiwix-tools for ARM64:

wget https://download.kiwix.org/release/kiwix-tools/kiwix-tools_linux-aarch64-3.8.2.tar.gz

tar -xzf kiwix-tools_linux-aarch64-3.8.2.tar.gz

sudo cp kiwix-tools_linux-aarch64-3.8.2/kiwix-* /usr/local/bin/

Update build-library.sh to point to NVMe:

sed -i 's|/home/pi/zims|/mnt/nvme/zims|g' /home/pi/build-library.sh

## 4.5 Configure Nginx

Update nginx to serve PDFs from NVMe and set up portal:

sudo nano /etc/nginx/sites-available/default

Update the root and pdfs location to point to NVMe paths:

root /var/www/html;

location /pdfs/ { alias /mnt/nvme/pdfs/; }

Copy portal to web root:

sudo cp /home/pi/index.html /var/www/html/index.html

sudo systemctl restart nginx

## 4.6 Configure Kiwix Service

Create systemd service pointing to NVMe library:

sudo nano /etc/systemd/system/kiwix.service

Contents:

[Unit]

Description=Kiwix Server

After=network.target

[Service]

User=pi

ExecStart=/usr/local/bin/kiwix-serve --library /mnt/nvme/zims/library.xml --port 8080

Restart=always

[Install]

WantedBy=multi-user.target

sudo systemctl enable kiwix

sudo systemctl start kiwix

## 4.7 Configure Relay Service

Copy relay files and create service (same as Pi 4):

sudo nano /etc/systemd/system/relay.service

Contents:

[Unit]

Description=SBS Relay Controller

After=network.target

[Service]

User=pi

WorkingDirectory=/home/pi

ExecStart=/usr/bin/python3 /home/pi/relay_server.py

Restart=always

[Install]

WantedBy=multi-user.target

sudo systemctl enable relay

sudo systemctl start relay

## 4.8 Rebuild Kiwix Library

~/build-library.sh

sudo systemctl restart kiwix

## 4.9 Configure Hotspot

Configure SBS hotspot (same SSID and IP as Pi 4):

sudo nmcli connection add type wifi ifname wlan0 con-name SBSHotspot autoconnect yes ssid SBS mode ap

sudo nmcli connection modify SBSHotspot \

802-11-wireless.mode ap \

802-11-wireless.band bg \

802-11-wireless.channel 6 \

ipv4.method shared \

ipv4.addresses 192.168.4.1/24 \

wifi-sec.key-mgmt wpa-psk \

wifi-sec.psk 'YourPasswordHere' \

connection.autoconnect yes

sudo nmcli connection up SBSHotspot

# 5. Phase 3 — OpenCPN Navigation Setup

## 5.1 Install OpenCPN

sudo apt install -y opencpn

Or for the latest version via flatpak:

sudo apt install -y flatpak

flatpak remote-add --if-not-exists flathub https://flathub.org/repo/flathub.flatpakrepo

flatpak install flathub org.opencpn.OpenCPN

## 5.2 Install OpenCPN Plugins

Essential plugins for offshore Pacific passage:

| Plugin | Purpose |
| --- | --- |
| dashboard_pi | Instrument displays — speed, heading, wind, depth |
| grib_weather_pi | GRIB weather file viewer for passage routing |
| ais_radar_pi | AIS target display and collision avoidance |
| watchdog_pi | Anchor drag alarm, arrival alarm, depth alarm |
| weatherfax_pi | HF weatherfax reception if SSB radio fitted |
| celestial_navigation_pi | Celestial navigation sight reduction |
| ocpn_draw_pi | Draw routes, boundaries, and waypoints |

Install plugins via OpenCPN: Options → Plugins → Search

## 5.3 Download Pacific Charts

All charts stored on NVMe at /mnt/nvme/charts/. Download using OpenCPN's built-in chart downloader:

OpenCPN → Options → Charts → Chart Downloader → Add Catalog

Priority chart folios for Pacific crossing:

| Chart Folio | Coverage | Source | Notes |
| --- | --- | --- | --- |
| NOAA RNC US West Coast | US West Coast | NOAA (free) | Start point — California to Mexico |
| NOAA ENCs | US Waters | NOAA (free) | Vector charts, better detail |
| NGA Charts Pacific | Pacific Ocean | NGA (free) | Open ocean passage charts |
| Mexico Pacific Coast | Baja to Panama | NGA/OpenCPN catalog | Coastal Mexico and Central America |
| Pacific Islands (COTP) | Hawaii, Polynesia, Micronesia | NGA (free) | Island landfall charts |
| New Zealand | NZ Waters | LINZ (free) | If routing via NZ |
| Australia | Australian Waters | AHS (free) | If routing via Australia |
| OpenSeaMap | Worldwide | OpenSeaMap (free) | Community maintained hazards overlay |

Download charts via terminal using OpenCPN chart downloader or manually:

mkdir -p /mnt/nvme/charts/noaa /mnt/nvme/charts/nga /mnt/nvme/charts/pacific

In OpenCPN set chart directory to /mnt/nvme/charts/ so all charts are on the NVMe.

## 5.4 Configure GPSd

GPSd feeds GPS data to OpenCPN and other applications:

sudo apt install -y gpsd gpsd-clients

sudo nano /etc/default/gpsd

Set the GPS device (MacArthur HAT GPS on UART0):

DEVICES="/dev/ttyAMA0"

GPSD_OPTIONS="-n"

sudo systemctl enable gpsd

sudo systemctl start gpsd

Test GPS is working:

cgps -s

In OpenCPN: Options → Connections → Add Connection

- Type: Serial

- Port: /dev/ttyAMA0 (or gpsd://localhost)

- Baud rate: 9600 (or as specified by your GPS module)

## 5.5 Configure NMEA 0183 Devices

The MacArthur HAT provides multiple UART ports for NMEA 0183 devices. Typical wiring:

| Device | MacArthur HAT Port | Baud Rate | Data Provided |
| --- | --- | --- | --- |
| VHF/AIS Receiver | UART1 (Serial1) | 38400 | AIS targets, VHF DSC |
| Simrad TP22 Autopilot | UART2 (Serial2) | 4800 | Heading, steering commands |
| Wind Transducer (N2K) | NMEA 2000 CAN bus | N/A | Wind speed, wind angle |
| GPS Module | UART0 (Serial0) | 9600 | Position, time, speed, course |

In OpenCPN add each NMEA source: Options → Connections → Add Connection → Serial → select port → set baud rate.

## 5.6 Configure OpenCPN for Offshore Use

Key settings to configure before departure:

- Options → Ships → set vessel length, beam, draft

- Options → Display → set night mode colors for watch-keeping

- Options → AIS → enable CPA/TCPA alarms (recommended: CPA 0.5nm, TCPA 30min)

- Options → Alarms → enable anchor drag alarm, set radius

- Dashboard plugin → add instrument windows for wind, depth, speed, GPS

- Set chart directory to /mnt/nvme/charts/

- Enable automatic chart updates when internet available

## 5.7 OpenCPN Remote Access via VNC

OpenCPN runs as a desktop application. Access it from any device on the SBS hotspot via VNC:

sudo apt install -y realvnc-vnc-server

sudo systemctl enable vncserver-x11-serviced

sudo systemctl start vncserver-x11-serviced

Connect from any device using VNC Viewer app to 192.168.4.1 — useful for tablets at the helm.

# 6. Phase 4 — MacArthur HAT & OpenPlotter Setup

## 6.1 Install OpenPlotter

OpenPlotter is the recommended software stack for the MacArthur HAT — it includes Signal K server, NMEA routing, and HAT configuration tools:

sudo apt install -y python3-openplotter-hat

Or install the full OpenPlotter suite:

wget -q https://raw.githubusercontent.com/openplotter/openplotter-install/master/install.sh | sudo bash

## 6.2 Configure MacArthur HAT

The MacArthur HAT is configured via OpenPlotter:

- OpenPlotter → HAT → detect MacArthur HAT

- Enable IMU (ICM-20948) for heading/heel/pitch data

- Enable barometer for atmospheric pressure

- Configure UART ports for each NMEA device

- Enable CAN bus for NMEA 2000 if using N2K wind transducer

## 6.3 Signal K Server

Signal K is the open marine data standard that ties all instruments together. OpenPlotter installs it automatically:

- Signal K server runs at http://192.168.4.1:3000

- All NMEA data is normalized into Signal K format

- OpenCPN, the boat dashboard, and other apps read from Signal K

- Signal K admin interface: http://192.168.4.1:3000/admin

## 6.4 NMEA 2000 Wind Transducer

If using a NMEA 2000 wind transducer (recommended: B&G WS310 or Garmin GWS 10):

- Connect N2K backbone to MacArthur HAT CAN bus connector

- OpenPlotter → CAN bus → enable and configure

- Signal K will automatically receive wind data from N2K backbone

- Wind data appears in OpenCPN dashboard and boat dashboard

## 6.5 Autopilot Integration (Simrad TP22)

The TP22 communicates via NMEA 0183:

- Connect TP22 NMEA output to MacArthur HAT UART2

- TP22 sends heading sentences (HDG, HDM, HDT)

- OpenCPN can send waypoint/route data back to TP22 for autopilot navigation

- Configure in OpenCPN: Options → Connections → enable output to TP22 port

# 7. Phase 5 — Unified Boat Dashboard

## 7.1 Overview

The boat dashboard is a custom web interface accessible from any phone, tablet, or laptop connected to the SBS hotspot. It provides a modern, simplified interface for managing the boat without needing to interact directly with OpenCPN or the command line.

The dashboard combines:

- Live instrument data from Signal K (wind, speed, heading, depth, GPS position)

- AIS target display via Signal K

- 12V relay control (lights, pumps, fans — same as current SBS portal)

- Quick access to SBS reference library (Kiwix content)

- Weather GRIB data display

- Anchor watch status

- System status (battery, UPS, disk space)

## 7.2 Architecture

| Component | Technology | Port |
| --- | --- | --- |
| Signal K server | Node.js (installed by OpenPlotter) | 3000 |
| Boat dashboard | Flask + HTML/CSS/JS | 8082 |
| SBS portal (Kiwix links) | Nginx | 80 |
| Kiwix content | kiwix-serve | 8080 |
| Relay controller | Flask | 8081 |
| OpenCPN VNC access | RealVNC | 5900 |

## 7.3 Dashboard Service Setup

The dashboard server will be created at /home/pi/dashboard_server.py and runs as a systemd service:

sudo nano /etc/systemd/system/dashboard.service

Contents:

[Unit]

Description=SBS Boat Dashboard

After=network.target signalk.service

[Service]

User=pi

WorkingDirectory=/home/pi

ExecStart=/usr/bin/python3 /home/pi/dashboard_server.py

Restart=always

[Install]

WantedBy=multi-user.target

The dashboard will be accessible at http://192.168.4.1:8082 and will be linked from the main SBS portal.

## 7.4 Signal K Data Integration

The dashboard reads live boat data from Signal K's REST and WebSocket APIs:

- Position: GET http://localhost:3000/signalk/v1/api/vessels/self/navigation/position

- Speed: GET http://localhost:3000/signalk/v1/api/vessels/self/navigation/speedOverGround

- Heading: GET http://localhost:3000/signalk/v1/api/vessels/self/navigation/headingTrue

- Wind: GET http://localhost:3000/signalk/v1/api/vessels/self/environment/wind

- Depth: GET http://localhost:3000/signalk/v1/api/vessels/self/environment/depth

- AIS targets: GET http://localhost:3000/signalk/v1/api/vessels

- Live streaming: WebSocket ws://localhost:3000/signalk/v1/stream

NOTE: Full dashboard build will be done as a separate session once all hardware is assembled and Signal K is confirmed working with actual NMEA devices.

# 8. UPS Configuration

## 8.1 GeeekPi UPS Gen 6 Setup

The UPS communicates via I2C (address 0x36) through pogo pins. Install the software:

pip3 install pi-ups-hat --break-system-packages

Or use GeeekPi's provided library:

git clone https://github.com/geeekpi/upsplus.git /home/pi/upsplus

cd /home/pi/upsplus && pip3 install -r requirements.txt --break-system-packages

## 8.2 Clean Shutdown Script

Create a script that monitors the UPS and shuts down cleanly when power is lost:

sudo nano /home/pi/ups_monitor.py

The script polls the UPS I2C registers, and when battery level drops below threshold triggers sudo shutdown now. GeeekPi provides example scripts in the upsplus repository.

sudo nano /etc/systemd/system/ups-monitor.service

Enable as a service so it starts on boot and monitors power continuously.

# 9. Complete Services Summary

All services that run on boot on the Pi 5:

| Service | Port | Auto-start | Description |
| --- | --- | --- | --- |
| nginx | 80 | Yes | Web portal & PDF serving |
| kiwix | 8080 | Yes | Offline content library (90GB+) |
| relay | 8081 | Yes | 12V relay controller |
| dashboard | 8082 | Yes | Unified boat dashboard |
| signalk | 3000 | Yes | Marine data server (OpenPlotter) |
| gpsd | 2947 | Yes | GPS data daemon |
| vncserver | 5900 | Yes | Remote desktop for OpenCPN |
| ups-monitor | — | Yes | UPS battery monitor & clean shutdown |
| SBSHotspot | — | Yes | Wi-Fi access point (SSID: SBS) |

# 10. Quick Reference

## Access URLs

| Resource | URL (Hotspot) | URL (Home Network) |
| --- | --- | --- |
| Main Portal | http://192.168.4.1 | http://sailboatserver.local |
| Kiwix Library | http://192.168.4.1:8080 | http://sailboatserver.local:8080 |
| Relay Controls | http://192.168.4.1:8081 | http://sailboatserver.local:8081 |
| Boat Dashboard | http://192.168.4.1:8082 | http://sailboatserver.local:8082 |
| Signal K | http://192.168.4.1:3000 | http://sailboatserver.local:3000 |
| SSH | ssh pi@192.168.4.1 | ssh pi@sailboatserver.local |

## Key Commands

| Command | Purpose |
| --- | --- |
| ~/switch-to-wifi.sh NetworkName | Switch to Wi-Fi for downloading content |
| ~/switch-to-hotspot.sh | Switch back to SBS hotspot |
| ~/build-library.sh && sudo systemctl restart kiwix | Rebuild Kiwix after adding ZIMs |
| sudo systemctl status kiwix nginx relay dashboard signalk | Check all services |
| df -h /mnt/nvme | Check NVMe disk space |
| cgps -s | Check GPS status |
| sudo shutdown now | Safe shutdown (wait for activity light to stop) |
| sudo reboot | Reboot |

## Relay Channel Map

| Channel | GPIO | Device |
| --- | --- | --- |
| CH1 | GPIO 26 | Cabin Lights |
| CH2 | GPIO 20 | Navigation Lights |
| CH3 | GPIO 21 | Anchor Light |
| CH4 | GPIO 5 | Bilge Pump |
| CH5 | GPIO 6 | Water Pump |
| CH6 | GPIO 13 | Ventilation Fan |
| CH7 | GPIO 19 | Instruments/Electronics |
| CH8 | GPIO 16 | Spare |

## Build Phases Checklist

| Phase | Status | Description |
| --- | --- | --- |
| Phase 1 | Pending hardware | Pi 5 base OS setup, NVMe formatting |
| Phase 2 | Pending hardware | Content migration from Pi 4 via rsync |
| Phase 3 | Pending hardware | OpenCPN install, chart download, GPS config |
| Phase 4 | Pending hardware | MacArthur HAT, OpenPlotter, Signal K, NMEA devices |
| Phase 5 | Pending hardware | Unified boat dashboard build |
| Phase 6 | Future | Physical boat installation, relay wiring, sea trials |

# 11. Notes & Known Issues

- Pi 5 runs significantly hotter than Pi 4 — active cooling is essential especially with HAT stack

- PETG is required for 3D printed enclosure — PLA warps in marine heat and UV

- Relay board GPIO 6 (CH5/Water Pump) was verified clear of UPS I2C conflict since UPS uses pogo pins

- All Kiwix content links in portal use onclick navigation to avoid Chrome cross-origin port blocking

- Hostname management: cloud-init config at /boot/firmware/user-data is the master hostname source — edit there first

- Network switching: single wlan0 radio cannot run hotspot and client simultaneously — use ethernet for maintenance

- Signal K and dashboard build to be completed as separate session once hardware is assembled

- OpenCPN chart downloads are large — use ethernet and Wi-Fi mode for initial download

- B&G WS310 recommended for wind transducer — native NMEA 2000, no converter needed, no moving parts

# 12. Phase 7 — Emergency Power Backup (LiFePO4)

The Pi runs on boat 12V normally. Phase 7 adds a dedicated LiFePO4 battery pack in the electrical cabinet that keeps the Pi, relays, and instruments running if main 12V power fails — without any switching glitch.

## 12.1 Topology

Boat 12V bus feeds a DC-DC charger which keeps the LiFePO4 pack topped up. The Pi and all loads run directly off the LiFePO4 pack output. When boat 12V drops the pack seamlessly takes over with zero interruption — no relay switching, no power glitch to the Pi.

## 12.2 Key Components

- LiFePO4 pack — 10-20Ah 12V (e.g. Dakota Lithium 10Ah or Renogy 12Ah). 10Ah gives ~8-10 hours runtime at Pi 5 load (~1A). 20Ah gives ~18-20 hours — enough to survive overnight with no charging.

- DC-DC charger — Victron Orion-Tr Smart 12/12-15A (isolated). Handles LiFePO4 charge profile correctly. Isolates the backup pack from the main bus so a dead house bank cannot backfeed through it. ~$80.

- Voltage monitor — small ADC board (e.g. INA219 or ADS1115) on Pi GPIO I2C bus. Reads pack voltage and triggers graceful shutdown when voltage drops below ~11.5V before pack is fully depleted.

- The GeeekPi UPS Gen 6 HAT (18650 cells, already in the stack) becomes a tertiary backup — it provides a final few minutes of runtime after the LiFePO4 pack is depleted to allow the safe shutdown script to complete.

## 12.3 Safe Shutdown Script

A systemd service polls pack voltage every 30 seconds. When voltage drops below 11.5V it logs the event, sends a toast to the portal (if reachable), and calls sudo shutdown now. Script location: /home/pi/voltage_monitor.py.

## 12.4 Layered Power Redundancy Summary

- Layer 1 — Normal: Boat 12V bus powers everything via DC-DC charger and LiFePO4 pack

- Layer 2 — 12V failure: LiFePO4 pack sustains Pi for 8-20 hours depending on pack size

- Layer 3 — LiFePO4 low (11.5V): Voltage monitor triggers graceful shutdown script

- Layer 4 — Final: GeeekPi UPS HAT (18650 cells) provides last few minutes for shutdown to complete

- Wi-Fi fallback: GL.iNet goes down → SBS hotspot activates, crew connects to 192.168.4.1

# 13. Phase 8 — GPIO Conflict Resolution & Physical Switches

The Waveshare Relay Board (B) and MacArthur HAT share one conflicting GPIO pin that must be resolved before first boot. Additionally, 5 physical manual override switches are added for critical loads using free GPIO pins.

## 13.1 GPIO Conflict — CH7 Relay vs MacArthur HAT 1-Wire

The MacArthur HAT uses GPIO 19 for 1-Wire sensors. The Waveshare Relay Board (B) default pin for CH7 (Instruments) is also GPIO 19. This conflict must be resolved by remapping CH7 to GPIO 17 using the relay board's jumper configuration.

### Hardware fix — move CH7 jumper to GPIO 17

- Locate the CH7 pin selection jumper on the Waveshare Relay Board (B)

- Move jumper from GPIO 19 pad to GPIO 17 pad

- Update relay_server.py: change RELAY_PINS = {1:26, 2:20, 3:21, 4:5, 5:6, 6:13, 7:19, 8:16} to RELAY_PINS = {1:26, 2:20, 3:21, 4:5, 5:6, 6:13, 7:17, 8:16}

### Full GPIO map after remap

- CH1 Cabin Lights — GPIO 26

- CH2 Navigation Lights — GPIO 20

- CH3 Anchor Light — GPIO 21

- CH4 Bilge Pump — GPIO 5

- CH5 Water Pump — GPIO 6

- CH6 Ventilation Fan — GPIO 13

- CH7 Instruments — GPIO 17 (REMAPPED from GPIO 19)

- CH8 Starlink Power — GPIO 16

## 13.2 Physical Manual Override Switches

Five physical toggle switches are added for critical loads — operable even if the Pi crashes or the portal is unreachable. Switches are wired to free GPIO pins with internal pull-ups enabled in software. Switch pulled to GND = ON, floating = OFF.

### Switch assignments

- SW1 Bilge Pump (CH4) — GPIO 4 — SAFETY CRITICAL

- SW2 Navigation Lights (CH2) — GPIO 22 — REQUIRED UNDERWAY

- SW3 Anchor Light (CH3) — GPIO 23

- SW4 Cabin Lights (CH1) — GPIO 24

- SW5 Instruments (CH7) — GPIO 27

### Wiring

- One side of each switch to the assigned GPIO pin

- Other side of each switch to GND (any ground pin on GPIO header)

- No external resistors needed — enable internal pull-ups in relay_server.py via lgpio.gpio_claim_input(h, pin, lgpio.SET_PULL_UP)

- Use waterproof panel-mount toggle switches — marine rated if possible (Blue Sea or Carling Technologies)

- Poll switch states in relay_server.py every 100ms — if switch state changes, trigger the corresponding relay

- Portal relay state display will reflect physical switch changes on next poll cycle