SailboatServer

Raspberry Pi 5 Complete Build Guide

Hardware Assembly · OpenPlotter · SignalK · OpenCPN · SBS Services

SV-Esperanza · Pacific Passage Edition · March 2026

| Hostname | SailboatServer |
| --- | --- |
| Base OS | OpenPlotter 4 Headless (Debian 12 Bookworm 64-bit) |
| Hardware | Raspberry Pi 5 8GB |
| Storage | 480GB NVMe SSD (PCIe Gen 3) |
| HAT Stack | GeeekPi UPS Gen 6 / Pi 5 / GeeekPi P33 NVMe+PoE / MacArthur HAT / Waveshare Relay (B) |
| Services | Kiwix · nginx · relay_server · SignalK · OpenCPN |
| Portal | http://sailboatserver.local |
| SignalK | http://sailboatserver.local:3000 |
| OpenCPN | Remote via VNC or AvNav browser |

PART 1 — HARDWARE ASSEMBLY

# 1. Parts List

Verify all parts are present before starting assembly.

| Item | Model | Notes |
| --- | --- | --- |
| Raspberry Pi 5 | Pi 5 8GB | Main compute board |
| NVMe+PoE HAT | GeeekPi P33 | PCIe Gen 3, NVMe M.2 slot |
| NVMe SSD | 480GB M.2 NVMe | Main storage for OS and ZIM files |
| UPS HAT | GeeekPi UPS Gen 6 | 18650 cells, pogo pins, sits under Pi 5 |
| Navigation HAT | MacArthur HAT | NMEA0183, NMEA2000, Seatalk1, 1-Wire |
| Relay Board | Waveshare RPi Relay Board (B) | 8-channel, lgpio, LOW-active |
| LiFePO4 Battery | 15Ah 12V | Emergency power backup |
| LiFePO4 Charger | 12V DC-DC LiFePO4 charger | 14.6V output, 3-5A |
| GL.iNet Router | GL-AR300M 300M | Wi-Fi AP, SSID: SV-Esperanza |
| Standoffs | M2.5 brass, assorted heights | Between each HAT layer |
| Thermal pad/paste | For Pi 5 CPU | Active cooling essential |
| USB-C cable | 5V/5A rated | Pi 5 power (if not using PoE) |

# 2. Pre-Assembly — Flash SD Card (Do This Now)

No NVMe USB adapter is needed. Flash OpenPlotter to a 32GB SD card on your Mac tonight, boot the Pi 5 from it tomorrow, then clone to the NVMe from within the running Pi. The SD card is only used for this initial setup — after cloning it is stored as a recovery backup.

## 2.1 Download OpenPlotter 4 Headless Image

- Go to: https://openplotter.readthedocs.io/latest/getting_started/downloading.html

- Download the OpenPlotter Headless edition (64-bit, Bookworm)

- Unzip the downloaded file — you will have a .img file

⚠ Make sure you download OpenPlotter v4 — it requires Debian 12 Bookworm. v3 will not work correctly on Pi 5.

## 2.2 Flash OpenPlotter to SD Card

- Insert the 32GB SD card into your Mac

- Open Raspberry Pi Imager

- Click 'Choose OS' → 'Use custom' → select the OpenPlotter .img file

- Click 'Choose Storage' → select the 32GB SD card

- Click the gear icon (⚙) and set: hostname = SailboatServer, enable SSH, username = pi, set password

- Click 'Write' and wait for flashing and verification to complete

- Eject SD card from Mac — set aside until hardware assembly is complete tomorrow

⚠ Do NOT set Wi-Fi in Raspberry Pi Imager — OpenPlotter manages its own network configuration. Setting it here can conflict.

# 3. Hardware Stack Assembly

Assemble in this exact order — bottom to top. The GeeekPi P33 must be directly on the Pi 5 to reach the PCIe FFC connector.

## 3.1 Stack Order (Bottom to Top)

| Position | Component | Connection Method | Notes |
| --- | --- | --- | --- |
| 1 — Bottom | GeeekPi UPS Gen 6 | Pogo pins to Pi 5 underside | Sits below Pi 5, no GPIO header needed |
| 2 | Raspberry Pi 5 8GB | — | Center of stack |
| 3 | GeeekPi P33 NVMe+PoE HAT | GPIO header + PCIe FFC cable | FFC cable must reach Pi 5 PCIe slot on board edge |
| 4 | MacArthur HAT | GPIO passthrough from P33 | NMEA/navigation connections on 3 sides |
| 5 — Top | Waveshare Relay Board (B) | GPIO passthrough from MacArthur | Screw terminals face out for wiring access |

## 3.2 Step-by-Step Assembly

- Install NVMe SSD into P33 HAT M.2 slot — secure with retention screw

- Attach standoffs to Pi 5 mounting holes — verify height clears P33 components

- Connect PCIe FFC ribbon cable to Pi 5 PCIe slot (on board edge, lift latch, insert blue side up, close latch)

- Mount P33 HAT onto Pi 5 GPIO header — connect other end of FFC cable to P33 (same orientation)

- Attach standoffs between P33 and MacArthur HAT layer

- Mount MacArthur HAT onto P33 GPIO passthrough header

- Attach standoffs between MacArthur HAT and Relay Board

- Before mounting Relay Board — remove CH5 jumper (GPIO19 conflicts with MacArthur HAT 1-Wire). CH6 (GPIO20) also conflicts with MacArthur HAT UART — leave jumper off for now, remap to GPIO17 via solder wire later.

- Mount Waveshare Relay Board on top

- Slide GeeekPi UPS Gen 6 under Pi 5 — align pogo pins with Pi 5 pads

- Install 18650 cells in UPS HAT if not already installed

⚠ The PCIe FFC cable cannot be bent sharply. Route it carefully through the case channel. Verify both latches are fully closed before powering on.

## 3.3 GPIO Conflict Resolution — Relay Board Jumper Mapping

IMPORTANT: The Waveshare Relay Board (B) ships with a non-standard jumper configuration that differs from the documentation. Actual jumper positions must be verified by physical inspection. The MacArthur HAT uses GPIO19 for 1-Wire and GPIO20 for UART — both conflict with relay channels.

⚠ Do not assume relay board jumpers match Waveshare documentation — verify each channel physically before software configuration.

| Channel | Load | GPIO | Status | Notes |
| --- | --- | --- | --- | --- |
| CH1 | Cabin Lights | GPIO 5 | ✅ Working | Verified during installation |
| CH2 | Navigation Lights | GPIO 6 | ✅ Working | Verified during installation |
| CH3 | Anchor Light | GPIO 13 | ✅ Working | Verified during installation |
| CH4 | Bilge Pump | GPIO 16 | ✅ Working | SAFETY CRITICAL — verified |
| CH5 | Unassigned | GPIO 19 | ⚠ Jumper removed | GPIO19 = MacArthur HAT 1-Wire — must not be used |
| CH6 | Water Pump | GPIO 20 | ⚠ Conflicts | GPIO20 = MacArthur HAT UART — remap to GPIO17 later |
| CH7 | Ventilation Fan | GPIO 21 | ✅ Working | Verified during installation |
| CH8 | Starlink Power | GPIO 26 | ✅ Working | Verified during installation |

CH6 future fix: desolder the GPIO20 trace on the CH6 jumper block and run a wire to GPIO17 (physical pin 11). GPIO17 is free with no conflicts. Update RELAY_PINS in relay_server.py to 7:17 when done.

## 3.4 Wiring — 12V Relay Outputs

The Waveshare Relay Board (B) is LOW-active — pulling the GPIO pin LOW turns the relay ON. Each channel has COM, NO (normally open), and NC (normally closed) terminals. Wire COM and NO for standard switched loads.

⚠ Use appropriately rated wire for each load. Bilge pump and water pump may draw 5-10A — use 14AWG minimum for these channels. Navigation and anchor lights are typically lower draw — 16-18AWG is sufficient.

## 3.5 Physical Manual Override Switches

Five toggle switches connect to free GPIO pins via the Relay Board's secondary GPIO header. Wire one side of each switch to the GPIO pin, other side to GND. No external resistors needed — software pull-ups are enabled.

| Switch | Load | GPIO Pin | Priority |
| --- | --- | --- | --- |
| SW1 | Bilge Pump (CH4) | GPIO 4 | SAFETY CRITICAL |
| SW2 | Navigation Lights (CH2) | GPIO 22 | Required underway |
| SW3 | Anchor Light (CH3) | GPIO 23 | Required at anchor |
| SW4 | Cabin Lights (CH1) | GPIO 24 | Comfort |
| SW5 | Ventilation Fan (CH7) | GPIO 27 | Convenience |

## 3.6 Power Connections

- Primary: Boat 12V bus → LiFePO4 charger → 15Ah LiFePO4 pack → DC-DC buck converter → Pi 5 USB-C (5V/5A)

- Relay loads: 12V directly from boat bus to relay COM terminals

- MacArthur HAT: 12V input from boat bus (HAT has onboard regulator)

- GL.iNet router: 12V from boat bus (router has onboard regulator)

⚠ The LiFePO4 pack sits permanently in circuit. When boat 12V drops the pack seamlessly takes over. Voltage monitor script triggers safe shutdown at 11.5V.

PART 2 — SOFTWARE SETUP

# 4. First Boot — OpenPlotter Headless

## 4.1 Initial Connection

- Insert the flashed SD card into Pi 5 SD card slot

- Power on Pi 5

- Wait 60-90 seconds for first boot to complete

- On your Mac, look for Wi-Fi network: openplotter

- Connect to openplotter Wi-Fi (password: 12345678)

- Open RealVNC Viewer and connect to: 10.10.10.1 — username: pi, password: raspberry

- Or open browser and go to: http://10.10.10.1 for the web interface

⚠ Default OpenPlotter headless credentials: SSID = openplotter, Wi-Fi password = 12345678, VNC/SSH user = pi, password = raspberry. Change ALL passwords immediately after first login.

## 4.2 Clone SD Card to NVMe — Do This First

Before doing anything else, clone the SD card to the NVMe SSD. The Pi is currently running from the SD card. After cloning it will boot permanently from the NVMe. The SD card is then stored as an emergency recovery backup.

- Verify the NVMe SSD is detected:

lsblk

You should see mmcblk0 (SD card) and nvme0n1 (NVMe SSD). If nvme0n1 is not visible, check that the PCIe FFC cable latches are fully closed on both ends.

- Clone SD card to NVMe — this takes 10-20 minutes:

sudo dd if=/dev/mmcblk0 of=/dev/nvme0n1 bs=4M status=progress

- Expand the NVMe partition to use the full 480GB:

sudo growpart /dev/nvme0n1 2

sudo resize2fs /dev/nvme0n1p2

- Verify full disk size is available:

df -h /

- Set boot order to NVMe first via raspi-config:

sudo raspi-config

In raspi-config: Advanced Options → Boot Order → NVMe/USB Boot first → Finish → reboot

- After reboot, verify you are running from NVMe (SD card still inserted is fine at this point):

findmnt / | grep nvme

Output should show /dev/nvme0n1p2. If it does — shut down, remove the SD card, and reboot from NVMe only.

sudo shutdown now

- Remove SD card and store as recovery backup — Pi 5 now boots from NVMe permanently

⚠ Keep the SD card. If the NVMe ever fails offshore it can be used to boot the Pi directly as emergency recovery.

- Change pi user password immediately: passwd

- Change OpenPlotter hotspot password via OpenPlotter Settings → Network

- Set hostname to SailboatServer: sudo hostnamectl set-hostname SailboatServer

- Update system: sudo apt update && sudo apt full-upgrade -y && sudo apt clean

- Reboot: sudo reboot

## 4.3 Pi 5 Hardware Notes — Differences from Pi 4

The Raspberry Pi 5 has several important differences from Pi 4 that affect GPIO, I2C, and device configuration. These were discovered during installation and are documented here to avoid confusion.

### GPIO Chip Numbering

Pi 5 uses gpiochip4 (pinctrl-rp1) for the 40-pin GPIO header, not gpiochip0 as on Pi 4. Any Python code using lgpio must open gpiochip4:

h = lgpio.gpiochip_open(4)   # Pi 5 — NOT gpiochip_open(0)

### I2C Bus

I2C must be enabled via raspi-config, not just dtparam in config.txt. After enabling, the GPIO header I2C appears as /dev/i2c-1. Buses i2c-11 and i2c-12 are internal Pi 5 system buses — ignore them. Enable with:

sudo raspi-config nonint do_i2c 0

Verified I2C devices on i2c-1:

| Address | Device | Notes |
| --- | --- | --- |
| 0x17 | GeeekPi UPS Gen 6 | Battery management, charge status |
| 0x68 | MacArthur HAT RTC | Real-time clock |

### 1-Wire Temperature Sensors

On Pi 5 the 1-Wire bus works natively without the w1-gpio overlay. Do NOT add dtoverlay=w1-gpio to config.txt — it will fail with a kernel error on Pi 5. The sensors are available immediately after boot at /sys/bus/w1/devices/.

Sensors installed on SV-Esperanza, mapped during installation 2026-03-08:

| Sensor Address | Location | Typical Range |
| --- | --- | --- |
| 28-000000240cbd | Exhaust temperature | Ambient to 200°C |
| 28-000000251764 | Water temperature | Ambient to 40°C |
| 28-0000008327eb | Engine bay temperature | Ambient to 80°C |
| 28-00000086defe | Cabin temperature | 15-35°C |

⚠ If sensors are not appearing at boot, check /sys/bus/w1/devices/ is present. The Pi 5 kernel handles 1-Wire on GPIO19 via the MacArthur HAT automatically — no overlay needed.

### Network on First Boot

The OpenPlotter headless image creates a Wi-Fi hotspot named 'openplotter' (password: 12345678). However the hotspot network range depends on the image edition:

- Headless edition: hotspot at 10.10.10.1, VNC to 10.10.10.1

- Desktop/starting editions: Network Manager creates hotspot in 10.42.0.x range — SSH to 10.42.0.1

If your Mac gets an IP in the 10.42.0.x range, SSH with: ssh pi@10.42.0.1, password: raspberry

⚠ passwd will fail with 'Authentication token manipulation error' on first SSH login. Use sudo passwd pi instead to change the password.

## 4.4 Network Configuration

Set a static IP on ethernet so the Pi is always reachable at a predictable address on the GL.iNet network. The Pi 5 uses Network Manager (nmcli) not dhcpcd:

sudo nmcli connection modify "Wired connection 1" \

ipv4.method manual \

ipv4.addresses 192.168.8.201/24 \

ipv4.gateway 192.168.8.1 \

ipv4.dns "192.168.8.1 8.8.8.8"

sudo nmcli connection up "Wired connection 1"

Disable the OpenPlotter hotspot from autostarting — keep it available as emergency fallback only:

sudo nmcli connection modify "OpenPlotter-Hotspot" connection.autoconnect no

Set hostname:

sudo hostnamectl set-hostname sailboatserver

From this point forward use 192.168.8.201 for all SSH and browser access. The hotspot can be re-enabled manually or via the portal System section.

# 5. OpenPlotter App Installation

## 5.1 Access OpenPlotter GUI via VNC

OpenPlotter is configured through a desktop GUI. Set up VNC to access it remotely:

sudo apt install -y realvnc-vnc-server

sudo systemctl enable vncserver-x11-serviced

sudo systemctl start vncserver-x11-serviced

Connect from Mac using RealVNC Viewer to 192.168.8.201. Login: pi / your password.

## 5.2 Install OpenPlotter Apps

In the VNC desktop, open OpenPlotter Settings. Install and update these apps:

| App | Purpose | Notes |
| --- | --- | --- |
| OpenPlotter SignalK Installer | Signal K server | Install first — required for everything else |
| OpenPlotter Serial | Serial/UART port management | Required for GPS and NMEA ports |
| OpenPlotter OpenCPN Installer | Chart plotter |  |
| OpenPlotter Dashboards | Grafana instrument displays |  |
| OpenPlotter Notifications | Alerts and alarms |  |
| OpenPlotter GPIO | GPIO integration |  |

⚠ The I2C and CAN Bus apps are not required — I2C is handled directly and there is no NMEA2000 on this vessel currently.

# 6. Signal K Configuration

## 6.1 Initial Setup

After installing Signal K via OpenPlotter SignalK Installer, access the admin UI at http://192.168.8.201:3000. Default credentials are admin/admin — change immediately to a secure password.

Signal K version installed: 2.22.1

## 6.2 Data Connections

In SignalK admin → Server → Data Connections, add the following connections:

| ID | Type | Device/Port | Baud | Purpose |
| --- | --- | --- | --- | --- |
| ESP32Sensors | NMEA0183 TCP Server | Port 10110 | — | ESP32 sensor node — speed, weather, bilge |
| GPS | NMEA0183 Serial | /dev/ttyOP_gps | 4800 | u-blox USB GPS receiver |
| vhf | NMEA0183 Serial | /dev/ttyOP_VHF | 38400 | VHF radio — AIS in, GPS position out |
| tp22 | NMEA0183 Serial | /dev/ttyOP_TP22 | 4800 | Simrad TP22 autopilot — bidirectional |

For the VHF and TP22 connections, set Output Events to: nmea0183out

Serial ports are bidirectional in SignalK 2.x — one connection handles both input and output on the same port.

⚠ Verify TCP port 10110 is listening after adding ESP32Sensors connection: sudo ss -tlnp | grep 10110

## 6.3 Signal K Plugins

In SignalK admin → Plugin Configuration, enable and configure:

| Plugin | Status | Configuration |
| --- | --- | --- |
| Convert Signal K to NMEA0183 | Enabled | Check: RMC, GGA, GLL, VTG, APB, XTE, HDM, MWV — throttle 1000ms each |
| Course Data Provider | Enabled | Default settings |
| Set System Time | Enabled | Sets Pi system clock from GPS time |
| Freeboard-SK | Enabled | Web-based instrument display |
| KIP | Enabled | KIP instrument panel server |

# 7. MacArthur HAT Configuration

## 7.1 UART Overlays

The MacArthur HAT uses UART2 and UART4 on Pi 5. These overlays are added to /boot/firmware/config.txt by the MacArthur HAT installation:

dtoverlay=uart2-pi5

dtoverlay=uart4-pi5

After reboot, verify the ports appear:

ls /dev/ttyAMA*

Expected output: /dev/ttyAMA10  /dev/ttyAMA2  /dev/ttyAMA4

## 7.2 UART Port Assignments

| UART | Device | GPIO Pins | Connected To | Baud | Direction |
| --- | --- | --- | --- | --- | --- |
| UART2 | ttyAMA2 → ttyOP_VHF | TX: GPIO0, RX: GPIO1 | VHF radio NMEA | 38400 | Bidirectional |
| UART4 | ttyAMA4 → ttyOP_TP22 | TX: GPIO8, RX: GPIO9 | Simrad TP22 autopilot | 4800 | Bidirectional |

## 7.3 Register Ports in OpenPlotter Serial

In the VNC desktop, open OpenPlotter Serial. For each port:

- Click ttyAMA2 → Name: VHF, Baud: 38400 → Add to SignalK

- Click ttyAMA4 → Name: TP22, Baud: 4800 → Add to SignalK

OpenPlotter Serial creates stable symlinks: /dev/ttyOP_VHF and /dev/ttyOP_TP22

## 7.4 GPS Configuration

The u-blox USB GPS receiver is detected as /dev/ttyACM0. OpenPlotter Serial creates a stable symlink /dev/ttyOP_gps. The GPS outputs standard NMEA0183 at 4800 baud.

GPS sentences verified working: GPRMC, GPVTG, GPGGA, GPGSA, GPGSV, GPGLL

GPS fix confirmed — navigation.position populating in SignalK Data Browser.

⚠ The u-blox GPS may take several minutes to acquire a fix indoors. Place antenna near a window or outside for first fix.

## 7.5 MacArthur HAT 1-Wire Sensors

Four DS18B20 temperature sensors are wired to the MacArthur HAT 1-Wire bus on GPIO19. On Pi 5 the 1-Wire bus works natively — do NOT add dtoverlay=w1-gpio to config.txt as it will fail.

| Sensor Address | Location | Alert Threshold |
| --- | --- | --- |
| 28-000000240cbd | Exhaust temperature | 90°C |
| 28-000000251764 | Water temperature | — |
| 28-0000008327eb | Engine bay | 80°C |
| 28-00000086defe | Cabin | 45°C |

Read all sensors: for sensor in /sys/bus/w1/devices/28-*/w1_slave; do echo "--- $(basename $(dirname $sensor)) ---"; cat $sensor | grep t= | awk -F't=' '{print $2/1000 " °C"}'; done

# 8. SailboatServer Services Installation

Install nginx, relay_server.py, and Kiwix on top of OpenPlotter as standard systemd services.

## 8.1 Install Dependencies

sudo apt install -y nginx i2c-tools python3-flask python3-lgpio

sudo pip3 install flask flask-cors paramiko --break-system-packages

⚠ flask-cors is required to allow the portal (port 80) to make API calls to the relay server (port 5000). Without it the browser will block all relay API requests with CORS errors.

## 8.2 Relay Server

Copy relay_server.py to /home/pi/relay_server.py. Key Pi 5 specific facts discovered during installation:

- Pi 5 GPIO header is on gpiochip4 (pinctrl-rp1), NOT gpiochip0. Use lgpio.gpiochip_open(4)

- The None guard in init_gpio is critical: if pin is not None: lgpio.gpio_claim_output(h, pin, 1)

- Relay board shipped with non-default jumper configuration — verify all channel GPIO assignments physically before running

- flask-cors must be installed and CORS(app) added to relay_server.py or the portal cannot reach the relay API from port 80

- Switch polling GPIO pins must not overlap with relay GPIO pins — SW4 was on GPIO24 which conflicted with CH6 relay, causing phantom relay triggers

GPIO conflicts discovered on SV-Esperanza — required jumper moves and overlay changes:

| GPIO | Conflict | Resolution |
| --- | --- | --- |
| GPIO19 | MacArthur HAT 1-Wire bus (DS18B20 sensors) | Remove CH5 jumper entirely |
| GPIO20 | MacArthur HAT UART2 TX (VHF radio) | Remove CH6 jumper entirely |
| GPIO21 | gpio-shutdown overlay (hardware shutdown switch) | Move CH7 jumper to GPIO18 |
| GPIO26 | gpio-poweroff overlay (HAT power module cutoff) | Move CH8 jumper to GPIO17 |

CH5 and CH6 were originally unassigned after jumper removal. New GPIO pins were soldered for all reassigned channels:

| Channel | GPIO | Load | Header Pin | Notes |
| --- | --- | --- | --- | --- |
| CH1 | GPIO5 | Cabin Lights | Pin 29 | Original jumper — unchanged |
| CH2 | GPIO6 | Navigation Lights | Pin 31 | Original jumper — unchanged |
| CH3 | GPIO13 | Anchor Light | Pin 33 | Original jumper — unchanged |
| CH4 | GPIO16 | Bilge Pump | Pin 36 | Original jumper — SAFETY CRITICAL |
| CH5 | GPIO25 | Water Pump | Pin 22 | Wire soldered — was unassigned |
| CH6 | GPIO24 | Ventilation Fan | Pin 18 | Wire soldered — was unassigned |
| CH7 | GPIO18 | Instruments | Pin 12 | Wire soldered — moved from GPIO21 |
| CH8 | GPIO17 | Starlink Power | Pin 11 | Wire soldered — moved from GPIO26 |

⚠ When adding physical override switches in future: SW4 was originally mapped to GPIO24 which is now CH6 relay output. SW4 must be reassigned to a free GPIO. Never assign a switch pin to the same GPIO as a relay output — the relay driving the pin low will be read as a switch press by the polling thread.

Hardware shutdown switch configuration — add to /boot/firmware/config.txt:

dtoverlay=gpio-shutdown,gpio_pin=21,active_low=1,gpio_pull=up

dtoverlay=gpio-poweroff,gpiopin=26,active_low=1

This uses the MacArthur HAT SeaTalk alternate switch input (GPIO21) to trigger a clean OS shutdown. GPIO26 signals the HAT power module to cut 12V supply after shutdown completes. Verify GPIO26 is held high during normal operation:

pinctrl get 26

Expected: op dh pd | hi — if lo, add gpio=26=pu to config.txt (known Pi 5 variance).

⚠ raspi-gpio is not supported on Pi 5. Use pinctrl instead: pinctrl get <pin>. pinctrl does not accept multiple pin arguments — query one pin at a time.

Create /etc/systemd/system/relay.service:

[Unit]

Description=SailboatServer Relay Control Server

After=sysinit.target

Before=network.target

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi

ExecStart=/usr/bin/python3 /home/pi/relay_server.py

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

⚠ relay.service must start before network.target (After=sysinit.target Before=network.target) to claim GPIO pins before they float. Without this, GPIO20 and others will be pulled low during boot causing relays to energize unexpectedly.

Add sudoers rules for hotspot control and system power:

sudo visudo -f /etc/sudoers.d/sailboatserver

Add these lines:

pi ALL=(ALL) NOPASSWD: /usr/bin/nmcli connection up OpenPlotter-Hotspot

pi ALL=(ALL) NOPASSWD: /usr/bin/nmcli connection down OpenPlotter-Hotspot

pi ALL=(ALL) NOPASSWD: /sbin/reboot

pi ALL=(ALL) NOPASSWD: /sbin/shutdown

## 8.3 Install Nginx and Deploy Portal

sudo apt install -y nginx

sudo systemctl enable nginx

sudo systemctl start nginx

Create portal directory structure:

sudo mkdir -p /var/www/html/docs/books

sudo mkdir -p /var/www/html/docs/engine

sudo mkdir -p /var/www/html/docs/sewing

sudo mkdir -p /var/www/html/docs/audio/be-the-captain

sudo mkdir -p /var/www/html/docs/video

sudo chown -R pi:pi /var/www/html

Copy portal to nginx root:

cp /home/pi/index.html /var/www/html/index.html

Verify portal at: http://192.168.8.201

## 8.4 Install Kiwix

cd ~

wget https://download.kiwix.org/release/kiwix-tools/kiwix-tools_linux-aarch64-3.8.2.tar.gz

tar -xzf kiwix-tools_linux-aarch64-3.8.2.tar.gz

sudo cp kiwix-tools_linux-aarch64-3.8.2/kiwix-serve /usr/local/bin/

## 8.5 Set Up NVMe Directories for Library

After NVMe HAT is installed and OS is cloned to NVMe, create library directories:

sudo mkdir -p /mnt/nvme/zims

sudo mkdir -p /mnt/nvme/charts

sudo chown -R pi:pi /mnt/nvme

## 8.6 Migrate ZIM Files from Pi 4

Transfer the ZIM library from the Pi 4 (~90GB) over the local network:

rsync -avz --progress pi@192.168.8.214:/home/pi/zims/*.zim /mnt/nvme/zims/

⚠ This transfer will take several hours. Use ethernet for maximum speed. Pi 4 IP: 192.168.8.214.

## 8.7 Build Kiwix Library and Create Service

kiwix-manage /mnt/nvme/zims/library.xml add /mnt/nvme/zims/*.zim

Create /etc/systemd/system/kiwix.service:

[Unit]

Description=Kiwix Server

After=network.target

[Service]

ExecStart=/usr/local/bin/kiwix-serve --port=8080 --library /mnt/nvme/zims/library.xml

Restart=always

User=pi

[Install]

WantedBy=multi-user.target

Enable all services:

sudo systemctl daemon-reload

sudo systemctl enable kiwix relay.service

sudo systemctl start kiwix relay.service

Configure the GL.iNet GL-AR300M to provide Wi-Fi access for crew devices on SV-Esperanza network.

| Router LAN IP | 192.168.8.1 |
| --- | --- |
| Pi ethernet IP | 192.168.8.x (DHCP from GL.iNet) |
| Wi-Fi SSID | SV-Esperanza |
| Wi-Fi password | Bund3r!8!8 |
| Root password | Bund3r!8!8 |

## 8.1 SSH Key Setup (Pi to Router)

ssh-keygen -t rsa -b 2048 -f ~/.ssh/id_rsa_router -N ""

ssh-copy-id -o HostKeyAlgorithms=+ssh-rsa \

-o PubkeyAcceptedAlgorithms=+ssh-rsa \

-i ~/.ssh/id_rsa_router.pub root@192.168.8.1

Add to ~/.ssh/config:

Host 192.168.8.1

IdentityFile ~/.ssh/id_rsa_router

HostKeyAlgorithms +ssh-rsa

PubkeyAcceptedAlgorithms +ssh-rsa

User root

# 9. SBS Hotspot (Fallback)

The Pi's built-in Wi-Fi provides a fallback hotspot if the GL.iNet goes down. OpenPlotter manages the hotspot — configure it via OpenPlotter Settings → Network.

| Hotspot SSID | SBS |
| --- | --- |
| Hotspot IP | 192.168.4.1 |
| Mode | Fallback — activate via switch script or portal button |

⚠ OpenPlotter dual-mode Wi-Fi means the Pi can connect to SV-Esperanza AND run the SBS hotspot simultaneously. Verify this works correctly after GL.iNet setup.

# 10. PCIe NVMe Configuration

Enable PCIe Gen 3 for maximum NVMe performance. Add to /boot/firmware/config.txt:

dtparam=pciex1

dtparam=pciex1_gen=3

Verify NVMe is detected after reboot:

lsblk

dmesg | grep pcie

# Should show PCIe Gen 3 negotiated

# 11. Verification Checklist

Complete this checklist after full assembly and software setup:

| Item | Command / URL | Expected Result |
| --- | --- | --- |
| NVMe detected | lsblk | nvme0n1 visible |
| PCIe Gen 3 | dmesg ¦ grep pcie | gen3 negotiated |
| UPS I2C | i2cdetect -y 1 | UPS address visible |
| Kiwix running | sudo systemctl status kiwix | active (running) |
| nginx running | sudo systemctl status nginx | active (running) |
| relay running | sudo systemctl status relay | active (running) |
| Signal K running | http://sailboatserver.local:3000 | Signal K dashboard loads |
| Portal loads | http://sailboatserver.local | SV-Esperanza portal loads |
| GPS data | Signal K data browser | navigation.position updating |
| Relay control | Portal → Boat Controls | 8 channel cards visible |
| Starlink status | Portal header | ONLINE/OFFLINE showing |
| AIS targets | OpenCPN chart | Nearby vessels showing |
| Kiwix library | http://sailboatserver.local:8080 | All ZIM books listed |

# 12. Key File Locations

| Portal (nginx) | /var/www/html/index.html |
| --- | --- |
| Portal (source) | /home/pi/index.html |
| Relay server | /home/pi/relay_server.py |
| ZIM library XML | /mnt/nvme/zims/library.xml |
| ZIM files | /mnt/nvme/zims/*.zim |
| Charts | /mnt/nvme/charts/ |
| Boot config | /boot/firmware/config.txt |
| SSH config | ~/.ssh/config |
| Router SSH key | ~/.ssh/id_rsa_router |
| kiwix.service | /etc/systemd/system/kiwix.service |
| relay.service | /etc/systemd/system/relay.service |

— End of Build Guide —

APPENDICES

# Appendix A — Current Offline Content Library

All content is served by Kiwix on port 8080 and accessible from the portal. Total library size is approximately 90GB. ZIM files are stored at /mnt/nvme/zims/. Access the full library browser at http://sailboatserver.local:8080

## A.1 Reference & Encyclopedias

| Title | Size | Description |
| --- | --- | --- |
| Wikipedia (English, no images) | ~51 GB | Full English Wikipedia, December 2025 snapshot |
| Wikibooks (English, all) | ~5.4 GB | Free textbooks and manuals on hundreds of subjects including navigation, seamanship, engineering |
| Wikispecies | ~3.4 GB | Taxonomy and species reference for all life on Earth — useful for fish ID, marine life |
| Wikivoyage (English, no images) | ~240 MB | Travel guides for every country, region, and port — customs, entry requirements, anchorages |

## A.2 Books & Literature (Project Gutenberg)

Five sections of the Project Gutenberg library are included, covering over 10,000 public domain books.

| Collection | Size | Highlights |
| --- | --- | --- |
| Gutenberg LCC-R (Recreation) | ~2.0 GB | Sailing classics, sport, outdoor pursuits — includes classic blue water cruising narratives |
| Gutenberg LCC-S (Science) | ~4.5 GB | Natural sciences, mathematics, biology, meteorology |
| Gutenberg LCC-T (Technology) | ~13 GB | Engineering, marine technology, navigation — largest and most useful for boat systems |
| Gutenberg LCC-U (Military Science) | ~1.3 GB | Military history and strategy |
| Gutenberg LCC-V (Naval Science) | ~1.3 GB | Naval history, seamanship, navigation classics — Bowditch, Lecky, and other historic sailing texts |

## A.3 Technical How-To (Stack Exchange Q&A)

| Site | Size | Value Offshore |
| --- | --- | --- |
| DIY Stack Exchange | ~2.1 GB | Boat and home repair Q&A — fiberglass, plumbing, electrical, structural |
| Mechanics Stack Exchange | ~339 MB | Engine and mechanical repair — diesel troubleshooting, alternators, drives |
| Engineering Stack Exchange | ~254 MB | Structural, mechanical, and electrical engineering Q&A |
| Chemistry Stack Exchange | ~416 MB | Useful for understanding antifouling, epoxy, fuel, water treatment |
| Ham Radio Stack Exchange | ~76 MB | Radio communications, antenna theory, emergency comms — SSB, VHF, AIS |
| Security Stack Exchange | ~440 MB | Cybersecurity and operational security reference |
| Outdoors Stack Exchange | ~142 MB | Wilderness survival, navigation, offshore fishing, weather reading |
| Sustainable Living Stack Exchange | ~27 MB | Off-grid power, water catchment, solar, composting |
| Astronomy Stack Exchange | ~196 MB | Celestial navigation reference, star identification, planetary motion |
| Biology Stack Exchange | ~422 MB | Marine biology, ecology, fish identification |
| Travel Stack Exchange | ~460 MB | Ports of entry, customs procedures, visa info for Pacific countries |
| Sports Stack Exchange | ~100 MB | General sports and fitness reference |

## A.4 Medical & Health

| Title | Size | Description |
| --- | --- | --- |
| Medical Sciences Stack Exchange | ~61 MB | Clinical medicine Q&A — dosing, symptoms, treatments |
| Zimgit Medicine | ~70 MB | Comprehensive offline medical reference guide for remote settings |
| Zimgit Water | ~20 MB | Water purification, treatment, and safety — watermaker maintenance |

## A.5 Survival & Emergency Preparedness

| Title | Size | Description |
| --- | --- | --- |
| Zimgit Post-Disaster Survival | ~644 MB | Comprehensive post-disaster reference — shelter, food, water, medical, communication |
| Zimgit Knots | ~28 MB | Knot tying reference with step-by-step illustrations — sailing and rescue knots included |

## A.6 Food & Provisioning

| Title | Size | Description |
| --- | --- | --- |
| Cooking Stack Exchange | ~237 MB | Recipes and cooking techniques — making do with limited provisions offshore |
| Zimgit Food Preparation | ~98 MB | Food preparation and preservation guide — canning, drying, fermenting |
| Homebrewing Stack Exchange | ~37 MB | Fermentation, brewing, and food preservation |
| Gardening Stack Exchange | ~925 MB | Growing food, tropical plants, sprouting — useful for extended passages |

## A.7 Repair Manuals

| Title | Size | Description |
| --- | --- | --- |
| iFixit | ~3.6 GB | Step-by-step repair guides with photos for electronics, appliances, devices — critical for fixing gear offshore |

# Appendix B — Recommended Additional Content

The following content is strongly recommended for a Pacific passage and is not yet on the server. Items are grouped by how to obtain them. All FREE items are legally available at no cost and should be downloaded before departure.

## B.1 Essential Boat Reference — Purchase Required

These books are under copyright but are the most important references to have aboard. Buy the eBook version, export or convert to PDF using Calibre (free app), and sideload via the instructions in Section B.6.

| Book | Author | Why It Matters | Where to Buy |
| --- | --- | --- | --- |
| Boatowner's Mechanical & Electrical Manual (4th Ed) | Nigel Calder | THE bible of boat systems — engines, electrical, plumbing, refrigeration, rigging, watermakers. If you only buy one book for offshore sailing it is this one. | Amazon Kindle or Google Play (~$30) |
| Marine Diesel Engines (3rd Ed) | Nigel Calder | Companion to the main Calder book focused entirely on diesel — bleeding, injectors, heat exchangers, raw water pumps, impellers. Essential when the engine acts up at sea. | Amazon Kindle (~$25) |
| Marine Diesel Engines Maintenance & Repair Manual | Jean-Luc Pallas | Rates each maintenance job by difficulty. The only book with detailed saildrive coverage — essential if your boat has a saildrive rather than a shaft drive. | Amazon (~$25) |
| The Voyager's Handbook (2nd Ed) | Beth Leonard | Comprehensive blue water cruising guide — watch schedules, provisioning, crew dynamics, heavy weather, making landfall. Written for Pacific-style passages. | Amazon Kindle (~$20) |
| Mariner's Handbook (NP100) | UK Hydrographic Office | Official reference for lights, buoys, tides, weather, currents, and passage planning. Used by professional mariners worldwide. | Amazon or UKHO direct (~$40) |
| Canvaswork and Sail Repair | Don Casey | The most practical sail and canvas repair book — grouped by repair type so you can find what you need fast when something tears offshore. | Amazon (~$20 used or Kindle) |
| The Sailmaker's Apprentice | Emiliano Marino | Over 700 detailed illustrations covering hand sewing, patches, tears, seams, rings, and making sails from scratch. The deepest reference for real sail repair underway. | Amazon Kindle (~$25) |
| Canvas for Cruisers | Julie Gifford | Covers fitting and fabrication of every interior and exterior canvas project likely needed offshore. Written by a circumnavigator who completed a Pacific crossing. | Amazon (~$40) |
| Be the Captain | James Evenson (Sailing Zingaro) | Offshore seamanship, leadership, and decision-making from 85,000 nautical miles. If you bought the hardcover, the PDF is included — check email from bethecaptain.com. | bethecaptain.com or Amazon |

## B.2 Free Marine Diesel Manuals — High Priority

MarineDieselBasics.com hosts nearly 3,000 free downloadable service manuals for most makes of diesel engines, cooling systems, fuel systems, transmissions, and saildrives. Having your specific engine's factory service manual offshore is invaluable — torque values, clearances, and diagrams for your exact engine that no generic book can provide.

- Go to: https://www.marinedieselbasics.com

- Find your engine make and model — download the complete factory service manual as PDF

- Also download manuals for: raw water pump, heat exchanger, fuel injectors, transmission/gearbox, and alternator if you have the model numbers

- Copy all manuals to /var/www/html/docs/engine/ on the server

⚠ Do this before departure. Knowing your exact engine model, serial number, and having its factory manual available offline has saved many offshore passages from becoming emergencies.

## B.3 Sewing & Canvas Resources — Free Online

Sailrite is the best free resource for canvas and sail repair. Their written guides are thorough enough to work from without video. Download these before departure and store in /var/www/html/docs/sewing/.

| Resource | Format | Where to Get It |
| --- | --- | --- |
| Sailrite How-To Guides | PDF / Web | https://www.sailrite.com/How-To — browse by project, save each guide as PDF from your browser |
| Sailrite Sail Repair Guide | PDF | Search: 'Sailrite sail repair guide PDF' — covers patches, tears, batten pockets, slides, hanks |
| Sailrite Canvas Projects Library | PDF | https://www.sailrite.com/Canvas-Projects — bimini, dodger, sail covers, bags, cushions |
| Sailrite Thread & Needle Reference | PDF | Thread weights, needle sizes, seam allowances for different canvas weights and sail materials |

## B.4 Hosting Be the Captain Audiobook on the Server

If you have the MP3 files they can be hosted on the server and streamed from any phone or tablet on the SV-Esperanza network using just a browser — no app needed.

- Create the audio directory:

sudo mkdir -p /var/www/html/docs/audio/be-the-captain

sudo chown -R pi:pi /var/www/html/docs/audio

- Copy MP3 files from your Mac:

scp ~/audiobooks/be-the-captain/*.mp3 \

pi@sailboatserver.local:/var/www/html/docs/audio/be-the-captain/

- Access from any device via browser:

http://sailboatserver.local/docs/audio/be-the-captain/chapter01.mp3

For a better experience, a full audiobook player page with chapter playlist, progress tracking, and controls styled to match the SBS portal can be built and linked from the portal home screen. Ask SailboatServer AI to build one when the server is running.

## B.5 Free Downloads — Get These Before Departure

All of the following are free, legal, and high value. Download, convert to PDF if needed, and copy to /var/www/html/docs/books/ on the server.

| Book | Source | Download URL |
| --- | --- | --- |
| American Practical Navigator (Bowditch) | US NGA — Public Domain | https://msi.nga.mil/Publications/APN |
| Offshore Cruising Encyclopedia (2nd Ed) — Dashew | Freely released by authors | https://www.setsail.com/free-books/ |
| Mariner's Weather Handbook — Dashew | Freely released by authors | https://www.setsail.com/free-books/ |
| Surviving the Storm — Dashew | Freely released by authors | https://www.setsail.com/free-books/ |
| Practical Seamanship — Dashew | Freely released by authors | https://www.setsail.com/free-books/ |
| Ship Captain's Medical Guide | UK Maritime & Coastguard Agency | Search: 'MCA Ship Captain Medical Guide PDF' |
| Navigation Rules & COLREGS | US Coast Guard — Public Domain | Search: 'USCG Navigation Rules PDF' |
| World Sailing Safety Recommendations | World Sailing | https://www.sailing.org/tools/documents/ |
| GMDSS Handbook | ITU — Public Domain | Search: 'ITU GMDSS Handbook PDF' |
| Your engine factory service manual | MarineDieselBasics.com | https://www.marinedieselbasics.com |
| Sailrite repair and canvas guides | Sailrite.com | https://www.sailrite.com/How-To |

## B.6 Additional Kiwix ZIM Files to Download

Available from https://download.kiwix.org/zim/ — download before departure and add to /mnt/nvme/zims/.

| ZIM File | Category | Why Add It |
| --- | --- | --- |
| physics.stackexchange.com | Stack Exchange | Sail trim theory, weather physics, celestial mechanics, navigation calculations |
| electronics.stackexchange.com | Stack Exchange | Marine electronics troubleshooting — chartplotters, AIS, VHF, autopilot repair |
| earthsciences.stackexchange.com | Stack Exchange | Oceanography, meteorology, tides — very useful for passage planning |
| sailing.stackexchange.com | Stack Exchange | Direct sailing Q&A if available in library |
| gutenberg remaining LCC sections | Project Gutenberg | You have R, S, T, U, V — consider A (General Works), G (Geography), K (Law) for port entry rules |
| zimgit-wikipedia-en-medicine | Zimgit | Medical Wikipedia subset — more detailed than zimgit-medicine for clinical reference |

## B.7 Pacific-Specific Chart & Cruising Guide Notes

Charts are handled separately by OpenCPN and are not ZIM files. Before departure ensure the following are loaded into OpenCPN on the NVMe:

- NOAA RNC/ENC charts — US West Coast and Hawaii — free from NOAA

- Pacific Islands charts — French Polynesia, Marquesas, Tuamotus, Society Islands — NV Charts or Navionics

- Mexico Pacific coast charts — Baja to Panama if routing south first

- OpenSeaMap overlay — free community-sourced hazard and anchorage data, import into OpenCPN

- World cruising routes GPX waypoints — many Pacific routes shared in the cruising community as GPX files importable into OpenCPN

## B.8 How to Add Content to the Server

All content lives under /var/www/html/docs/ and is served directly by nginx. Suggested folder structure:

sudo mkdir -p /var/www/html/docs/books

sudo mkdir -p /var/www/html/docs/engine

sudo mkdir -p /var/www/html/docs/sewing

sudo mkdir -p /var/www/html/docs/audio/be-the-captain

sudo mkdir -p /var/www/html/docs/video

sudo chown -R pi:pi /var/www/html/docs

Copy files from your Mac via scp:

scp ~/Downloads/nigel_calder_mechanical.pdf \

pi@sailboatserver.local:/var/www/html/docs/books/

scp ~/Downloads/engine_manuals/*.pdf \

pi@sailboatserver.local:/var/www/html/docs/engine/

scp ~/Downloads/sailrite_guides/*.pdf \

pi@sailboatserver.local:/var/www/html/docs/sewing/

Access any file from any device on SV-Esperanza network:

http://sailboatserver.local/docs/books/nigel_calder_mechanical.pdf

http://sailboatserver.local/docs/engine/yanmar_3gm30_service.pdf

After adding content, update the portal index.html with a Reference Library section containing links organized by category — books, engine, sewing, audio — so crew can find everything without needing to remember file paths.

# Appendix C — ESP32-WROOM Sensor Node

A single ESP32-WROOM reads multiple boat sensors and publishes data to SignalK via NMEA0183 over TCP. Real sensor data appears on every phone, tablet, and chart plotter on the SV-Esperanza network with no dedicated displays required.

⚠ The ESPHome API block has been removed — the ESP32 runs standalone without Home Assistant. All data flows to SignalK via TCP NMEA0183 on port 10110.

## C.1 Sensor Summary

| Sensor | Data Output | NMEA Sentence | Status |
| --- | --- | --- | --- |
| Paddlewheel sender | Speed through water (knots) | $IIVHW | Build now |
| BME280 | Barometric pressure, air temp, humidity | $IIMDA + $IIXDR | Build now |
| Bilge water contact | Bilge wet/dry alert | HA alert only | Build now |
| Optocoupler + alternator W | Engine RPM, engine hours | $IIRPM | Parts on order |
| INA219 + fuel sender | Fuel tank level % | $IIXDR | Parts on order |
| ZMPT101B | Shore power AC voltage alert | HA alert only | Parts on order |

## C.2 Parts List

| Part | Purpose | Cost | Notes |
| --- | --- | --- | --- |
| ESP32-WROOM dev board | Main controller | On hand | 38-pin version |
| BME280 module | Pressure / temp / humidity | On hand | I2C, 3.3V |
| Paddlewheel sender | Speed through water | On hand | 2-wire reed switch type |
| Bilge water contact | Bilge alert | On hand | Float switch or bare contacts |
| Optocoupler module | Engine RPM isolation | ~$2 | PC817 or similar — on order |
| INA219 module | Fuel sender voltage | ~$3 | I2C — on order |
| ZMPT101B module | AC shore power monitor | ~$4 | Must have transformer on board — on order |
| 240Ω resistor 1/4W | Fuel sender voltage divider | <$1 |  |
| 1kΩ resistor 1/4W | RPM optocoupler input protection | <$1 |  |

⚠ Do NOT use the YX850 module for AC sensing — it is a DC power failover switch, not a voltage sensor. The ZMPT101B is the correct part for 120V AC monitoring.

## C.3 GPIO Pin Assignments

ADC2 pins cannot be used for analog reads when Wi-Fi is active. All analog sensors use ADC1 pins only. GPIO6-11 are reserved for internal flash — never use.

| GPIO | Function | Type | Notes |
| --- | --- | --- | --- |
| GPIO4 | Paddlewheel pulse | Digital interrupt | ADC2 — safe for digital |
| GPIO21 | I2C SDA | I2C | Shared bus: BME280 + INA219 |
| GPIO22 | I2C SCL | I2C | Shared bus: BME280 + INA219 |
| GPIO25 | Engine RPM | Digital interrupt | Via optocoupler |
| GPIO23 | Bilge water sensor | Digital input | Moved from GPIO26 — same right-side rail as other sensors |
| GPIO32 | Shore power (ZMPT101B) | ADC1 ✅ | Safe with Wi-Fi active |
| GPIO34 | Fuel sender (INA219) | ADC1 ✅ | Input-only pin |
| GPIO36 | Spare analog | ADC1 ✅ | Reserved |
| GPIO39 | Spare analog | ADC1 ✅ | Reserved |

## C.4 Wiring

### Paddlewheel Sender

- Wire 1 → GPIO4 | Wire 2 → GND | Internal pull-up enabled — no resistor needed

### BME280

- VCC → 3.3V | GND → GND | SDA → GPIO21 | SCL → GPIO22

### Bilge Water Sensor

- Wire 1 → GPIO23 | Wire 2 → GND | Contacts bridging GPIO to GND triggers alert

⚠ GPIO23 is on the same physical rail as GPIO21/22 (I2C) and GPIO4 (paddlewheel) for clean wiring. GPIO26 was vacated.

### Fuel Sender via INA219 (parts on order)

- INA219: VCC → 3.3V, GND → GND, SDA → GPIO21, SCL → GPIO22

- 3.3V → 240Ω resistor → INA219 V+ → fuel sender wire 1 → GND

- INA219 V- → GND

⚠ Tap into the existing sender wire alongside the original fuel gauge — do not replace it. The ESP32 reads in parallel so the analog gauge remains as a backup.

### Engine RPM via Optocoupler (parts on order)

- Optocoupler input+ → alternator W terminal via 1kΩ series resistor

- Optocoupler input- → GND | Optocoupler signal output → GPIO25

⚠ The 1kΩ resistor and optocoupler provide galvanic isolation protecting the ESP32 from alternator voltage spikes. Never connect the alternator W terminal directly to the ESP32.

### Shore Power via ZMPT101B (parts on order)

- VCC → 3.3V | GND → GND | OUT → GPIO32

- AC input terminals → shore power hot and neutral via inline 500mA fuse

⚠ Use caution with 120V AC wiring. If not comfortable with AC connections have a marine electrician make them.

## C.5 ESPHome Config Files

Two files live in the same ESPHome config folder. All configurable values are in the substitutions block at the top of the YAML — that is the only section that should need editing.

| File | Purpose |
| --- | --- |
| sv_esperanza_sensors.yaml | Main config — sensor definitions, calibration values, NMEA output |
| nmea_client.h | TCP client — SignalK connection and sentence transmission |

The two values in nmea_client.h (SIGNALK_HOST and SIGNALK_PORT) must match the corresponding substitutions in the YAML. ESPHome substitutions do not pass into .h files at compile time so these are the only duplicated values in the config.

## C.6 SignalK TCP Connection Setup

- Open SignalK admin: http://sailboatserver.local:3000/admin

- Server → Connections → Add

- Type = TCP | Direction = Input | Protocol = NMEA0183 | Port = 10110

- Save and restart SignalK

- After flashing ESP32, verify in SignalK Data Browser:

- navigation.speedThroughWater — from $IIVHW

- environment.outside.pressure — from $IIMDA

- environment.outside.temperature — from $IIXDR

- environment.outside.humidity — from $IIXDR

## C.7 Calibration

### Paddlewheel — Pulses Per Nautical Mile

- Motor in flat calm water at steady speed, note GPS SOG

- Check pulses/sec in ESPHome logs

- pulses_per_nm = (pulses_per_sec / GPS_SOG) × 3600

- Average results from 2-3 speeds, update paddle_pulses_per_nm, reflash via OTA

### Engine RPM

- Run engine at known RPM from existing tachometer

- Note raw pulse count in ESPHome logs

- rpm_multiplier = known_RPM / raw_count

- For accuracy use calibrate_linear with idle, cruise, and WOT data points

### Fuel Sender

- Fill tank — note INA219 bus_voltage in logs → set as fuel_voltage_full

- Run to known low level — note voltage → interpolate fuel_voltage_empty

- Update substitutions and reflash via OTA

## C.8 Flashing the ESP32

Flash via the Pi over USB for first flash. Future updates use OTA over Wi-Fi.

### Setup — on the Pi

- pip3 install esphome --break-system-packages

- mkdir -p ~/esp32 and copy sv_esperanza_sensors.yaml + nmea_client.h there

- Plug ESP32 into Pi via USB data cable

- cd ~/esp32 && esphome run sv_esperanza_sensors.yaml

- Select the /dev/ttyUSB0 option (not the GPS at /dev/ttyACM0)

### OTA Updates

- esphome run sv_esperanza_sensors.yaml

- Select Over The Air (192.168.8.50) — no USB needed

⚠ ESPHome via Homebrew on Mac causes PlatformIO Python dependency conflicts (littlefs, fatfs). Always flash from the Pi to avoid this.

## C.9 nmea_client.h — Implementation Notes

The TCP client uses lwip native sockets directly rather than Arduino WiFiClient. This is required because ESPHome's IDF framework does not include WiFiClient.h. Key implementation details:

- Include: lwip/sockets.h — not sys/socket.h (causes namespace conflict with esphome::socket)

- Use lwip_socket(), lwip_connect(), lwip_send(), lwip_close() — not bare socket() calls

- DNS not used — SIGNALK_HOST must be a raw IP address (no hostname resolution)

- BME280 I2C address: 0x76 (fixed, no SDO/ADDR pin on this module)

- Reconnect interval: 5 seconds — drops sentences during disconnect, reconnects automatically

— End of Appendices —